Addendum 001

Invitation to Bid HNS 26-11

NOTE: Contractors are not authorized to visit the property before or after the bid walk, without being accompanied by the City’s Rehabilitation Specialist.

Documents included in Package:

- Instruction to Bidders

- Specs by Location/Trade (Scope of Work)

- Subcontractor Certifications (if applicable)

- Floor Plan / Site Drawing (if applicable)

Bid Walk & Bid Opening:

| Project Address: 4418 Oak Dr – Safe Home, Lead, and Healthy Homes |  |
| --- | --- |
| Bid Walk: May 14, 2026 @ 9:30 AM |  |
| Bid Opening: June 4, 2026 @ 2:00 PM |  |
| Client Name: Sheila Linney |  |
| Project Manager: Devin Smith | Contact Number: (704) 589-2130 |

Bid Walk and Bidding Instructions:

All bid walks are mandatory.

If you are going to be late the policy is the following:

Contact me BEFORE the start time if you are going to be late. If you are going to be more than 10 minutes late, we will proceed without you and you will not be permitted to bid.

The day of a bid walk the best way to reach me is at   (704) 589-2130

Bids must be received by the date, time and place specified. All others will be considered non responsive and disqualified.

The Bids will be opened at 600 East Trade St. immediately following the above due date and time.

Company Acknowledgement: The undersigned, having become thoroughly familiar with the terms, conditions, limitations, and provisions of the housing improvement work to be performed at _4418 Oak Dr__ to be funded through the City of Charlotte Neighborhood & Business Services, in addition, having fully inspected the site in all particulars, hereby proposes and agrees to fully perform the work within the time stated and in strict accordance with the proposed contract documents including furnishing of any and all labor and materials, and to do all work required to complete said Work in accordance with the advised respective contractual, for the sum of money: All labor, materials, services and equipment necessary for the completion of the Work shown on the Drawings and in the Specifications: Dollars_($ )_ Written total

Specs Dated: 5/26/2026 Number of Pages: 16 Total

Addenda # 1 Dated: Number of Pages:

Addenda # 2 Dated: Number of Pages:

Project Schedule: Minimum Start Date -

Completion Deadline:

Please Print and Sign:

Company Name/Firm:

Authorized Representative Name:

Signature: Date:

Requirements For Bidders

The City awards rehabilitation bids to the lowest responsive and responsible bidder. A responsible bidder for the safe home program is one who:

- Is a licensed general contractor in the State of North Carolina

- Has a Renovate, Repair & Paint Certification (http://www2.epa.gov/lead/renovation-repair-and-painting-program)

- Is not listed on a local, state or federal debarment list

- Carries an appropriate amount of insurance

- Can provide references verifying the contractor has completed work of a similar scope in a good workmanship like manner or successfully completed prior work for the Safe Home program. Referenced work must have been completed in one year or less from date of this invitation to bid.

A responsive bidder must:

- Submit all requested documentation on time

- Meet the above requirements for responsibility at the time of bid submittal

- Have the capacity to meet the required schedule for the project

- Existing rehab projects contracted by the contractor must be on schedule

The City reserves the right to waive any minor informalities or irregularities, which do not go to the heart of the bid submittal or prejudice other offers, or to reject, for good and compelling reasons, any or all bid submittals.

Rehabilitation Specialist – Devin Smith City of Charlotte

Neighborhood and Business Services 600 E. Trade St.

Charlotte, NC 28202

PH: ( 7 0 4 ) 5 8 9 - 2 1 3 0

Fax:

Instructions to Bidders

Explanations to Bidders

Any explanation desired by a Contractor regarding the meaning or interpretation of the advertisement for bids, drawings, specifications, etc., must be requested in writing to the Rehabilitation Specialist with sufficient time allowed for a reply to each Contractor before the submission of their bids. Any interpretation made will be in the form of an addendum to the invitation for bids, drawings, specifications, etc., and will be furnished to all prospective Contractors. The Contractor must acknowledge any revision to the bid documents in the space provided on the bid form and it must be submitted with their bid package.

Preparation of Bids

Bids shall be submitted on the forms furnished, or copies thereof, and must be manually signed. If erasures or other changes appear on the forms, the person signing the bids must initial each erasure or change

- The Contractor’s Bid Proposal must be properly executed and submitted on the form provided. Bids by Contractors must be received by the Rehabilitation Specialist at the time and place specified on the “Invitation to Bid”

- No bid will be considered unless all individual work items on the Bid Form including any addendums are priced. The sum of all work items must equal to the Total Bid Amount.

- Unless called for, alternate bids will not be considered

- Modification of bids already submitted will be considered if received at the office designated in the invitation for bids by the time set for opening of bids

Submittal of Bids

- Sealed bids will be submitted to the Safe Home Program Office as directed in the invitation to bid prior to or at the appointed bid opening time

- Bid will be time stamped on the date of delivery

- Each bid must be submitted in a sealed envelope bearing on the outside the name of the Contractor, the name of the project for which the bid is submitted, and the date and time of the bid opening

- All bids submitted must be typed or written in ink and signed by the Contractor's designated representative

- Failure to include all forms may result in rejection of a bid

- Required Bid Package Forms:

  - Scope of Work

  - Addenda Acknowledgement

  - Itemized Work Sheet

  - Subcontractor Certifications (if applicable)

Bid Validation

- All approved bids will be valid for 60 days after the bid opening date

HOUSING AND NEIGHBORHOOD SERVICES

http://housing.charlottenc.gov | 600 E. Trade Street | Charlotte, NC

City of Charlotte Housing and Neighborhood Services

600 E. Trade Street Charlotte, NC 28202

704-336-7600	5/26/2026

Location:	1 - General Requirements	Approx. Wall SF: 0	Ceiling/Floor SF:	0

##### Spec #	Spec	Quantity	Units	Unit Price	Total Price

##### Trade:	1	General Requirements

##### 010	Asbestos Abatement Scope

See attached scope. An allowance to complete the scope of work attached.

##### Custom Lead Abatement Scope

See attached scope. An allowance to complete the scope of work attached.

#### Location Total:

Location:	2 - Exterior	Approx. Wall SF: 0	Ceiling/Floor SF:	0

##### Spec #	Spec	Quantity	Units	Unit Price	Total Price

Trade:	4	Site Work

Address: 4418 Oak Drive	Unit:  Unit 01

016

Exterminate Termites

Provide and perform complete termite extermination services for all affected and surrounding accessible areas as necessary to eliminate active infestation and reduce the risk of recurrence. Treatment shall include drilling and injection of an approved professional-grade termiticide at manufacturer-recommended intervals around the foundation, including treatment down to the footer where required. Drill and patch concrete, masonry, pavement, slabs, or other finished surfaces as necessary to complete treatment and maintain proper spacing and coverage. All termite treatment work shall be performed by a properly licensed pest control contractor in accordance with all applicable laws, regulations, and industry standards. Contractor shall provide written documentation of treatment performed and warranty terms, including a minimum one-year transferable warranty with optional annual renewal, and may offer extended coverage where available.

Location:	2 - Exterior	Approx. Wall SF: 0	Ceiling/Floor SF:	0

##### Spec #	Spec	Quantity	Units	Unit Price	Total Price

##### Trade:	32	Roof/Chimney/Gutters/Downspouts

##### 067	Tear Off & Reroof Shingles

Remove and dispose of existing roofing, inspect and repair roof deck (include 160 sq.ft. of decking replacement in bid) and install new 30 year architectural shingles. Provide the homeowner with a minimum of 3 color options. Include ice and water shield at all roof edges, felt paper, shingle over ridge vent, boots, flashing and all necessary accessories.

Repair/replaced all damaged or decayed fascia, rafters, ceiling joists, or other roof/attic area framing.

##### Trade:	1500	Mechanical

- C15500  REPAIR HEATING

Existing gas pack HVAC unit is currently providing cooling only. Heating system is inoperative and requires full diagnostic evaluation, with repair or replacement as necessary to restore proper operation.

#### Location Total:

Location:	3 - Interior	Approx. Wall SF: 0	Ceiling/Floor SF:	0

##### Spec #	Spec	Quantity	Units	Unit Price	Total Price

Trade:	20	Flooring

- 0231

Floor System Repair

In the hallway, living room/great room, kitchen, den, den bathroom, and laundry closet, the contractor shall remove and properly store or dispose of all fixtures, furnishings, appliances, cabinetry, countertops, plumbing fixtures, and other items necessary to complete the work. This includes both built-in and free-standing components such as toilets, vanities, bathtubs, cabinets, countertops, and appliances.

Existing fixtures and appliances shall be reinstalled upon completion unless damaged, unsafe, or otherwise unsuitable for reuse. If replacement is required, the contractor shall provide the homeowner with a minimum of three comparable replacement selections for approval prior to purchase or installation.

Remove and dispose of any damaged, deteriorated, or unsafe components of the floor assembly and crawl space support system as necessary to restore structural integrity. Repairs may include, but are not limited to, replacement of deteriorated band joists, floor joists, masonry piers, subflooring, and structural supports. New floor joists may consist of 2" x 8" framing spaced 16" on center, together with new plywood or tongue-and-groove subflooring as required. Replace all damaged or missing floor insulation and install new insulation where required to comply with current building code requirements. Install drop girders where necessary to properly support the floor system. Install floor insulation in accordance

Location:	3 - Interior	Approx. Wall SF: 0	Ceiling/Floor SF:	0

##### Spec #	Spec	Quantity	Units	Unit Price	Total Price

##### Trade:	20	Flooring

with all applicable building codes and manufacturer requirements.

Additionally, identify and repair any observable plumbing leaks and correct any visible electrical code violations located within the crawl space.

Install waterproof interlocking vinyl plank flooring with a minimum 25-year manufacturer’s warranty. All materials and installation methods shall strictly comply with the manufacturer’s specifications and recommendations. The contractor shall provide the homeowner with a minimum of three to five flooring pattern and color selections from approved manufacturers prior to installation.

The contractor shall prepare, level, and otherwise condition the subfloor as necessary to satisfy manufacturer installation requirements and ensure a proper finished product.

Install appropriate matching transition strips wherever the new flooring meets adjacent flooring materials.

Install painted or stained wood quarter-round molding along all finished perimeters to complete the flooring installation.

| Trade: | 21 | HVAC |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 7 | 0135 | HALLWAY BATH FAN |  |  |  |
|  |  | Replace ceiling mounted Energy Star rated exhaust fan. Fan shall be quieter than 2 sones and shall move air at 100 cfm. |  |  |  |
|  |  | Proper installation of metal vent and roof or wall cap/damper assembly is included. |  |  |  |
|  |  | Installation of all necessary wiring for fan and switch is included. |  |  |  |
|  |  | All materials and work shall comply with the Electrical and Mechanical Codes. |  |  |  |
|  |  | Repair all ceiling and wall damage caused by the installation. |  |  |  |
| Trade: | 22 | Plumbing & Fixtures |  |  |  |
| 8 | 0214 | Replace Hallway Bathroom Tub/Shower Valve and Trim |  |  |  |

Location:	3 - Interior	Approx. Wall SF: 0	Ceiling/Floor SF:	0

##### Spec #	Spec	Quantity	Units	Unit Price	Total Price

##### Trade:	22	Plumbing & Fixtures

Contractor to diagnose and repair the existing leaking tub spout/shower valve assembly to eliminate the constant drip, while minimizing demolition and preserving the existing tile surround to the greatest extent possible. Replace tub/shower trim components, including tub spout, handles, escutcheons, and shower head, as required to complete the repair in accordance with applicable Plumbing Code and Construction Standards. If valve replacement or access requirements necessitate removal of existing tile surround materials, remove the existing surround and install a new white Swan or approved equal three-piece fiberglass or FRP surround. Existing bathtub to remain in place and be protected during construction.

Contractor to provide all required trim, transition materials,

drywall repair, caulking, painting, and finish work necessary to leave the area in complete, finished condition with no exposed or unfinished surfaces.

##### Trade:	23	Electrical

- 0148	Light Fixture Replace

Replace hall bathroom wall mounted 4 bulb strip, UL approved, LED light fixture with shade and lamps.

##### 0153	GFCI Receptacle 20 AMP

Install a flush-mounted, ground fault circuit interrupter (GFCI) duplex receptacle and cover plate in the hallway bathroom. Fish wire and repair any tear-out as needed.

##### 0170	SMOKE AND CARBON MONOXIDE DETECTORS

Install UL approved ceiling mounted smoke and heat detectors permanently hard wired into outlet boxes with battery backups in all bedrooms and outside of all sleeping areas.

Detector in the hall way shall be a combination CO/smoke detector.

All detectors shall be interconnected so that when any one detector goes off, all other detectors also go off.

Installation shall comply with all requirements of the Electrical Code SMOKE AND CARBON MONOXIDE DETECTORS

##### 0172	Demo or Terminate Wires in Box

Completely eliminate the exposed, abandoned wires behind the oven. If unable to completely demo, terminate the existing

wires in a code-approved junction box.

##### 0185	Oven Circuit

Reroute oven circuit within walls so that all conductors are concealed or otherwise protected from physical damage.

Terminate conductors in an approved junction box or receptacle box with proper cable clamps/strain-relief and a listed cover plate, located in an accessible position within the cabinet.

Location:	3 - Interior	Approx. Wall SF: 0	Ceiling/Floor SF:	0

##### Spec #	Spec	Quantity	Units	Unit Price	Total Price

##### Trade:	23	Electrical

Test circuit operation, verify grounding and polarity, and correct any related electrical deficiencies discovered during this work.

##### 0186	REPLACE RECEPTACLES, SWITCHES AND COVER PLATES

Replace all existing receptacles, switches, device cover plates, and blank covers throughout the entire house (including the exterior, attic, and crawlspace).

Replace all two prong receptacles with three prong receptacles. Install GFCI breakers on all two wire receptacle circuits.

Install GFCI/AFCI breakers on all bedroom receptacle circuits. Label outlets per Code requirements.

Receptacles in Code defined wet or damp locations shall be GFCI protected.

Scope includes repair of any wall or ceiling damage.

##### Trade:	34	Doors & Windows

- 0108	EXTERIOR DOOR HARDWARE

Replace rear and side entry hardware with lever handle entry set and single keyed deadbolt.

NOTE: Double keyed deadbolts are not permitted. All entry locksets shall be keyed alike.

Deadbolt keepers shall be installed with long screws that attach securely to framing (new prehung doors equipped with reinforced metal plates on the back side of the jamb are also acceptable in lieu of long screws).

The finish of new hardware shall match existing hardware unless home owner selects otherwise.

Trade:	35	Wall Coverings/Ceilings/Trim

- 0244

Termite Damage

Inspect and selectively open affected walls, floors, ceilings, and structural components as necessary to determine the full extent of termite damage, moisture intrusion, rot, and any related deterioration, including damage to adjacent framing and concealed structural elements. Areas of concern include, but are not limited to, the wall between the den and kitchen, the wall between the kitchen and living room, ceiling beam in living room, and the front exterior wall assembly.

Location:	3 - Interior	Approx. Wall SF: 0	Ceiling/Floor SF:	0

##### Spec #	Spec	Quantity	Units	Unit Price	Total Price

##### Trade:	35	Wall Coverings/Ceilings/Trim

Remove and replace all damaged, deteriorated, or structurally compromised materials as necessary to restore the structure to a safe and code-compliant condition. Repairs may include, but are not limited to, replacement of wall studs, top and bottom plates, beams, headers, joists, subflooring, sheathing, blocking, and other framing or structural components. All repairs shall comply with current building codes and accepted construction standards.

Repair or replace any insulation, drywall, plaster, wall finishes, ceiling finishes, trim, flooring, and related finish materials impacted by the work. New materials shall match existing finishes, textures, and appearance as closely as reasonably possible. Where replacement of fixtures, cabinetry, countertops, appliances, or other components becomes necessary due to damage or removal during construction, the contractor shall provide the homeowner with a minimum of three comparable replacement selections for approval prior to installation.

The contractor shall coordinate and perform any temporary disconnection, removal, relocation, or adjustment of electrical, plumbing, or HVAC components required to complete the work. All affected systems shall be properly restored in a safe, fully functional, and code-compliant condition upon completion.

Replace any damaged or missing insulation within walls, floors, or ceilings as required to comply with applicable building code requirements.

##### 0264

The contractor shall obtain all required permits, coordinate inspections, protect adjacent areas and finishes during construction, and maintain a clean and safe work site throughout the project. Upon completion, the contractor shall remove all debris, construction materials, and waste related to the work.

##### Ceiling Repair Throughout the House

Inspect all rooms for cracked, sagging, stained, or otherwise damaged drywall/plaster ceilings, including prior patch areas and visible water damage.

Remove loose, soft, or delaminated material in affected areas and open up as needed to confirm that underlying framing is sound and dry.

Replace damaged drywall/plaster with new materials of

Location:	3 - Interior	Approx. Wall SF: 0	Ceiling/Floor SF:	0

##### Spec #	Spec	Quantity	Units	Unit Price	Total Price

##### Trade:	35	Wall Coverings/Ceilings/Trim

matching thickness; tape, mud, and sand to a smooth finish, including re-texturing to blend with existing surfaces as closely as practical.

Prime all repaired areas, and any stained surfaces, with an appropriate stain-blocking primer.

Apply finish paint to all repaired ceilings (and to logical break points such as entire ceilings as agreed), using

owner-approved colors and sheens, and feather into adjacent areas for a uniform appearance.

Repair or reinstall any trim, fixtures, or hardware disturbed during the work, perform paint touch-ups on affected trim, and leave rooms clean and ready for use.

| Trade: | 36 | Insulation |  |  |
| --- | --- | --- | --- | --- |
| 18 | 0266 | ATTIC INSULATION |  |  |
|  |  | Remove the homeowner's items stored in the attic, along with the existing attic insulation, and dispose of both properly. |  |  |
|  |  | Install a wooden or plywood dam around the access hatch high enough to hold back R-38 insulation. |  |  |
|  |  | Install continuous foam insulation around the perimeter of the panel where it rests on the casing. Glue an R-38 insulation batt cut to size on the top of the panel. |  |  |
|  |  | Install ventilation baffles per the Building Code. |  |  |
|  |  | Install insulation depth markers readily visible from the attic access hatch. |  |  |
|  |  | Blow in insulation to increase R value to R-38. |  |  |
|  |  | Attach insulation certificate to rafter at attic access, positioned to be easily seen. |  |  |
| Trade: | 38 | Stairs/Steps |  |  |
| 19 | 0286 | Folding Attic Stairs |  |  |
|  |  | Replace the utility folding stairway and install or adjust framing as needed to ensure proper installation in accordance with the manufacturer's specifications. Install casing that matches the existing trim in the room. Prime and paint the door and casing. Additionally, install a sealed, manufactured insulation cover over the attic stairs to achieve a minimum R-10 insulation value. |  |  |
|  |  |  | Location Total: |  |

#### Unit Total for 4418 Oak Drive, Unit Unit 01:		 Address Grand Total for 4418 Oak Drive:

Bidder:

## Scope of Work for Lead Hazard Control

Address:	4418 Oak Dr	Date: 5/26/2026

All work described must be performed by a North Carolina state-certified lead firm, and a state lead permit is required. All work must comply with all applicable federal, state, and local regulations. Provide a separate price for each line item and add the total of line items at the end of the scope. Transfer the lead scope total to the lead control line item in the main scope of work.

| Lead Hazard (from LIRA) | Method of Control and Scope of Work | Number | Unit Cost | Line-Item Cost |
| --- | --- | --- | --- | --- |

| Side A through Side D – beige wood fascias, soffits, crown moldings and trim boards (Note: It appears some of the fascia boards have been randomly replaced.) | Remove and properly dispose of all gutters, downspouts, and crown molding. Remove all existing electrical, supports, and light fixtures, and store them safely for reinstallation. Install new white K-type .027 gauge seamless aluminum gutter to service the roof with sufficient pitch to new downspouts, install to construction manual specifications. Install 4” corrugated black pipe with the proper connectors to each downspout. Pipe length shall be long enough to ensure roof runoff flows away from the foundation. All damaged, loose, or rotted material associated with the required repair shall be removed and replaced. Cover fascia, soffits (including vents/install new where needed), trim boards, and drip boards with Tyvek and aluminum or vinyl. Reroute and support all existing cabling. Reinstall all light fixtures and ensure they are operational. |  |  |  |
| --- | --- | --- | --- | --- |

| Door A1 (to Room 1) – beige wood door casings, header, jambs and stops | Remove entire existing door assembly (including threshold); dispose of properly. Install a new Energy Star-rated pre-hung metal exterior door and threshold. Owner's choice of door style, color (door and threshold), and hardware finish. Insulate cavity. Install a single-cylinder deadbolt and lever-handled entry set door hardware, keyed alike. New casing and shoe molding will match the existing. Apply primer and topcoat. Inspecting from the crawlspace during the bid walk is recommended; this installation includes replacing all damaged, loose, or rotted material associated with the required repair. |  |  |  |
| --- | --- | --- | --- | --- |
| Room 4 – Door D1 – beige wood door casings, header, jambs and stops | Make smooth and operable, scrape loose paint, and re-paint. Owner's choice of color. |  |  |  |
| Room 1 – Window A1, Window A2 and Window A3 – beige vinyl mini-blinds (lead contained within the matrix of the vinyl) | Remove, properly dispose of, and replace with new. Owner's choice of color and style. |  |  |  |
| Room 2 – Window C1 – beige vinyl mini-blind (lead contained within the matrix of the vinyl) | Remove, properly dispose of, and replace with new. Owner's choice of color and style. |  |  |  |

| Room 3 – Closet D1 – beige vinyl vertical blind door (lead contained within the matrix of the vinyl) | Remove, properly dispose of, and replace with new. Owner's choice of color and style. |  |  |  |
| --- | --- | --- | --- | --- |
| Room 8 – Window A1 and Window D1 – beige vinyl miniblinds (lead contained within the matrix of the vinyl) | Remove, properly dispose of, and replace with new. Owner's choice of color and style. |  |  |  |
| Throughout – elevated lead dust levels | Complete specialized cleaning throughout the house. |  |  |  |

| All contractor requirements | The contractor is responsible for all project requirements, including but not limited to: All activities required by the City’s COVID-19 Precautions for Residential Housing Rehabilitation Activities policy. Obtaining all permits required. Said permits shall include all items in this scope of work. Provide temporary toilet facilities from the job start until the completion of work. Provide AS MANY roll-off dumpsters as needed without damaging the site. Collect construction debris using dust control methods. Remove dumpsters and repair any evidence of dumpster use. Contractor may haul debris away daily using dump trailers or trucks. |  |  |  |
| --- | --- | --- | --- | --- |

Total Bid for Lead Scope $

## Scope of Work for Asbestos Hazard Control

Address: 4418 Oak Dr	Date: 5/26/2026

All work disturbing asbestos containing materials (ACM) must be performed in compliance with all applicable federal, state, and local regulations. Work in the rehab scope that reasonably can be anticipated to disturb ACM is listed below. The contractor is responsible for identifying all activities that disturb ACM and for the proper removal and disposal of all ACM disturbed while working on this project.

If all disturbed ACM materials will fit into a single asbestos waste or glove bag, contractor's workers who are properly trained under OSHA Construction Standard Asbestos Regulations 29 CFR 1926.1101 may remove and dispose of the ACM using safe work practices.

If all disturbed ACM materials will not fit into a single asbestos waste or glove bag, or if workers are not properly trained, the contractor shall hire a state certified asbestos abatement firm to remove and dispose of the disturbed ACM materials.

Provide a separate price for each line item and add the total of line items at the end of the scope. Transfer the asbestos scope total to the asbestos control line item in the main scope of work.

| Asbestos Hazard (from asbestos report) | Work activities that will or may disturb identified Asbestos Containing Material (ACM) | Line Item Cost |
| --- | --- | --- |
| INTERIOR - Green Vinyl Flooring (UNDER Carpet) - Approximately 280 square feet of Green Vinyl Flooring (UNDER Carpet) on the Den floor (back left room). | Work that may disturb ACM includes, but is not limited to: *Cutting, drilling, sawing, sanding, or structurally altering in a way that the material could become friable. Air sampling clearance must be completed after disturbing ACM. |  |

Total Bid for Asbestos Scope: $