Invitation to Bid HNS 26-11

NOTE: Contractors are not authorized to visit the property before or after the bid walk, without being accompanied by the City’s Rehabilitation Specialist.

Documents included in Package:

- Instruction to Bidders

- Specs by Location/Trade (Scope of Work)

- Subcontractor Certifications (if applicable)

- Floor Plan / Site Drawing (if applicable)

Bid Walk & Bid Opening:

| Project Address: 4418 Oak Dr – Safe Home, Lead, and Healthy Homes |  |
| --- | --- |
| Bid Walk: May 14, 2026 @ 9:30 AM |  |
| Bid Opening: May 26, 2026 @ 2:00 PM |  |
| Client Name: Sheila Linney |  |
| Project Manager: Devin Smith | Contact Number: (704) 589-2130 |

Bid Walk and Bidding Instructions:

All bid walks are mandatory.

If you are going to be late the policy is the following:

Contact me BEFORE the start time if you are going to be late. If you are going to be more than 10 minutes late, we will proceed without you and you will not be permitted to bid.

The day of a bid walk the best way to reach me is at   (704) 589-2130

Bids must be received by the date, time and place specified. All others will be considered non responsive and disqualified.

The Bids will be opened at 600 East Trade St. immediately following the above due date and time.

Company Acknowledgement: The undersigned, having become thoroughly familiar with the terms, conditions, limitations, and provisions of the housing improvement work to be performed at _4418 Oak Dr__ to be funded through the City of Charlotte Neighborhood & Business Services, in addition, having fully inspected the site in all particulars, hereby proposes and agrees to fully perform the work within the time stated and in strict accordance with the proposed contract documents including furnishing of any and all labor and materials, and to do all work required to complete said Work in accordance with the advised respective contractual, for the sum of money: All labor, materials, services and equipment necessary for the completion of the Work shown on the Drawings and in the Specifications: Dollars_($ )_ Written total

Specs Dated: 4/1/2026 Number of Pages: 15 Total

Addenda # 1 Dated: Number of Pages:

Addenda # 2 Dated: Number of Pages:

Project Schedule: Minimum Start Date -

Completion Deadline:

Please Print and Sign:

Company Name/Firm:

Authorized Representative Name:

Signature: Date:

Requirements For Bidders

The City awards rehabilitation bids to the lowest responsive and responsible bidder. A responsible bidder for the safe home program is one who:

- Is a licensed general contractor in the State of North Carolina

- Has a Renovate, Repair & Paint Certification (http://www2.epa.gov/lead/renovation-repair-and-painting-program)

- Is not listed on a local, state or federal debarment list

- Carries an appropriate amount of insurance

- Can provide references verifying the contractor has completed work of a similar scope in a good workmanship like manner or successfully completed prior work for the Safe Home program. Referenced work must have been completed in one year or less from date of this invitation to bid.

A responsive bidder must:

- Submit all requested documentation on time

- Meet the above requirements for responsibility at the time of bid submittal

- Have the capacity to meet the required schedule for the project

- Existing rehab projects contracted by the contractor must be on schedule

The City reserves the right to waive any minor informalities or irregularities, which do not go to the heart of the bid submittal or prejudice other offers, or to reject, for good and compelling reasons, any or all bid submittals.

Rehabilitation Specialist – Devin Smith City of Charlotte

Neighborhood and Business Services 600 E. Trade St.

Charlotte, NC 28202

PH: ( 7 0 4 ) 5 8 9 - 2 1 3 0

Fax:

Instructions to Bidders

Explanations to Bidders

Any explanation desired by a Contractor regarding the meaning or interpretation of the advertisement for bids, drawings, specifications, etc., must be requested in writing to the Rehabilitation Specialist with sufficient time allowed for a reply to each Contractor before the submission of their bids. Any interpretation made will be in the form of an addendum to the invitation for bids, drawings, specifications, etc., and will be furnished to all prospective Contractors. The Contractor must acknowledge any revision to the bid documents in the space provided on the bid form and it must be submitted with their bid package.

Preparation of Bids

Bids shall be submitted on the forms furnished, or copies thereof, and must be manually signed. If erasures or other changes appear on the forms, the person signing the bids must initial each erasure or change

- The Contractor’s Bid Proposal must be properly executed and submitted on the form provided. Bids by Contractors must be received by the Rehabilitation Specialist at the time and place specified on the “Invitation to Bid”

- No bid will be considered unless all individual work items on the Bid Form including any addendums are priced. The sum of all work items must equal to the Total Bid Amount.

- Unless called for, alternate bids will not be considered

- Modification of bids already submitted will be considered if received at the office designated in the invitation for bids by the time set for opening of bids

Submittal of Bids

- Sealed bids will be submitted to the Safe Home Program Office as directed in the invitation to bid prior to or at the appointed bid opening time

- Bid will be time stamped on the date of delivery

- Each bid must be submitted in a sealed envelope bearing on the outside the name of the Contractor, the name of the project for which the bid is submitted, and the date and time of the bid opening

- All bids submitted must be typed or written in ink and signed by the Contractor's designated representative

- Failure to include all forms may result in rejection of a bid

- Required Bid Package Forms:

  - Scope of Work

  - Addenda Acknowledgement

  - Itemized Work Sheet

  - Subcontractor Certifications (if applicable)

Bid Validation

- All approved bids will be valid for 60 days after the bid opening date

HOUSING AND NEIGHBORHOOD SERVICES

http://housing.charlottenc.gov | 600 E. Trade Street | Charlotte, NC

City of Charlotte Housing and Neighborhood Services

600 E. Trade Street Charlotte, NC 28202

704-336-7600	4/1/2026

Location:

1 - General Requirements

Approx. Wall SF: 0

Ceiling/Floor SF:	0

Address: 4418 Oak Drive	Unit:  Unit 01

##### Spec #	Spec	Quantity	Units	Unit Price	Total Price

##### Trade:	1	General Requirements

##### 010	Asbestos Abatement Scope

See attached scope. An allowance to complete the scope of work attached.

##### Custom Lead Abatement Scope

See attached scope. An allowance to complete the scope of work attached.

#### Location Total:

Location:

2 - Exterior

Approx. Wall SF: 0

Ceiling/Floor SF:	0

##### Spec #	Spec	Quantity	Units	Unit Price	Total Price

##### Trade:	4	Site Work

##### 016	Exterminate Termites

Exterminate for termites. Drill and patch foundation and pavement where necessary. Includes treatment throughout the house. Exterminator must be licensed and provide a 5 year guarantee with an optional yearly renewable warranty for the homeowner.

| Trade: | 21 | HVAC |  |  |
| --- | --- | --- | --- | --- |
| 4 | 0131 | HEATING AND AIR CONDITIONING EQUIPMENT INSTALL |  |  |
|  |  | Remove all existing HVAC equipment and dispose of properly. |  |  |
|  |  | Use Manual S to select the proper equipment for the house. Provide Manual J and S calculations and design specifications for the selected equipment to the rehab specialist for review and approval. |  |  |
|  |  | 1. Gas Pack Install the approved gas pack at the existing location. |  |  |

Location:

2 - Exterior

Approx. Wall SF: 0

Ceiling/Floor SF:	0

##### Spec #	Spec	Quantity	Units	Unit Price	Total Price

##### Trade:	21	HVAC

Gas fired equipment shall have minimum AFUE of 90. Installation includes inspection and replacement as necessary of gas lines and connections and exhaust venting.

Heat pumps and air conditioning equipment shall have a minimum SEER of 14. Installation includes new linesets.

Install the equipment complete with all devices, connections, refrigerant, and accessories necessary to produce a fully functioning HVAC system meeting all the requirements of the NC Mechanical and Electrical Codes as well as the Construction Specifications.

Install a new digital programmable thermostat.

Scope of work includes all repairs to the structure and finish surfaces that are necessary due to the removal of the old system and the installation of the new system.

Scope of work includes start up and testing of the new system, including demonstrating to the customer how to operate the thermostat and change the return air filter.

Provide rehab specialist with all installation manuals, user manuals, and warranty documentation.

##### Trade:	32	Roof/Chimney/Gutters/Downspouts

- 067	Tear Off & Reroof Shingles

Remove and dispose of existing roofing, inspect and repair roof deck (include 160 sq.ft. of decking replacement in bid) and install new 30 year architectural shingles. Provide the homeowner with a minimum of 3 color options. Include ice and water shield at all roof edges, felt paper, shingle over ridge vent, boots, flashing and all necessary accessories.

Repair/replaced all damaged or decayed rafters, ceiling joists, or other roof/attic area framing.

#### Location Total:

Location:

3 - Interior

Approx. Wall SF: 0

Ceiling/Floor SF:	0

##### Spec #	Spec	Quantity	Units	Unit Price	Total Price

##### Trade:	20	Flooring

##### 0231	Floor System Repair

In the hallway, living room/great room, kitchen, den, den bathroom, and laundry closet, remove all fixtures, both built-in and free-standing. Dispose of any damaged or unsafe components of the floor assembly. This work may involve the installation of 2" x 8" joists spaced 16" apart on center, as well as plywood or tongue-and-groove subflooring.

Install waterproof interlocking vinyl plank flooring that comes

Location:

3 - Interior

Approx. Wall SF: 0

Ceiling/Floor SF:	0

##### Spec #	Spec	Quantity	Units	Unit Price	Total Price

##### Trade:	20	Flooring

with a minimum 25-year manufacturer's warranty. The installation must strictly follow the manufacturer's specifications.

The contractor is responsible for preparing the subfloor as needed to meet the manufacturer's installation requirements.

Install appropriate matching transition pieces wherever the new flooring meets other types of flooring.

Finally, install painted or stained wood quarter-round along all perimeters to complete the installation.

Provide the homeowner with a choice of three to five pattern selections from approved manufacturers.

Throughout the crawl space, replace all deteriorated band joists and masonry piers, ensuring that the floor is insulated in accordance with code. Install drop girders as necessary.

Additionally, identify and repair any plumbing leaks and electrical violations in the crawl space.

| Trade: | 21 | HVAC |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 7 | 0135 | HALLWAY BATH FAN |  |  |  |
|  |  | Replace ceiling mounted Energy Star rated exhaust fan. Fan shall be quieter than 2 sones and shall move air at 100 cfm. |  |  |  |
|  |  | Proper installation of metal vent and roof or wall cap/damper assembly is included. |  |  |  |
|  |  | Installation of all necessary wiring for fan and switch is included. |  |  |  |
|  |  | All materials and work shall comply with the Electrical and Mechanical Codes. |  |  |  |
|  |  | Repair all ceiling and wall damage caused by the installation. |  |  |  |
| Trade: | 22 | Plumbing & Fixtures |  |  |  |
| 8 | 0214 | Replace Hallway Bathroom Tub/Shower Valve and Trim |  |  |  |
|  |  | Remove the existing tub and shower valve and trim kit and replace with new per the Plumbing Code and the Construction Standards. |  |  |  |
|  |  | Access to the valve will be through the back side of the tub wall. DO NOT REMOVE TUB SURROUND FINISH MATERIALS. |  |  |  |
|  |  | Scope includes the installation of a removable access panel on the back side wall or bathroom wall to fit neatly and tightly to the wall surface. |  |  |  |

Location:

3 - Interior

Approx. Wall SF: 0

Ceiling/Floor SF:	0

##### Spec #	Spec	Quantity	Units	Unit Price	Total Price

##### Trade:	23	Electrical

##### 0148	Light Fixture Replace

Replace hall bathroom wall mounted 4 bulb strip, UL approved, LED light fixture with shade and lamps.

##### 0153	GFCI Receptacle 20 AMP

Install a flush-mounted, ground fault circuit interrupter (GFCI) duplex receptacle and cover plate in the hallway bathroom. Fish wire and repair any tear-out as needed.

##### 0170	SMOKE AND CARBON MONOXIDE DETECTORS

Install UL approved ceiling mounted smoke and heat detectors permanently hard wired into outlet boxes with battery backups in all bedrooms and outside of all sleeping areas.

Detector in the hall way shall be a combination CO/smoke detector.

All detectors shall be interconnected so that when any one detector goes off, all other detectors also go off.

Installation shall comply with all requirements of the Electrical Code SMOKE AND CARBON MONOXIDE DETECTORS

##### 0172	Demo or Terminate Wires in Box

Completely eliminate the exposed, abandoned wires behind the oven. If unable to completely demo, terminate the existing

wires in a code-approved junction box.

##### 0185	Oven Circuit

Reroute oven circuit within walls so that all conductors are concealed or otherwise protected from physical damage.

Terminate conductors in an approved junction box or receptacle box with proper cable clamps/strain-relief and a listed cover plate, located in an accessible position within the cabinet.

Test circuit operation, verify grounding and polarity, and correct any related electrical deficiencies discovered during this work.

##### 0186	REPLACE RECEPTACLES, SWITCHES AND COVER PLATES

Replace all existing receptacles, switches, device cover plates, and blank covers throughout the entire house (including the exterior, attic, and crawlspace).

Replace all two prong receptacles with three prong receptacles. Install GFCI breakers on all two wire receptacle circuits.

Install GFCI/AFCI breakers on all bedroom receptacle circuits. Label outlets per Code requirements.

Receptacles in Code defined wet or damp locations shall be GFCI protected.

Scope includes repair of any wall or ceiling damage.

Location:

3 - Interior

Approx. Wall SF: 0

Ceiling/Floor SF:	0

##### Spec #	Spec	Quantity	Units	Unit Price	Total Price

##### Trade:	23	Electrical

##### Trade:	34	Doors & Windows

- 0108	EXTERIOR DOOR HARDWARE

Replace rear and side entry hardware with lever handle entry set and single keyed deadbolt.

NOTE: Double keyed deadbolts are not permitted. All entry locksets shall be keyed alike.

Deadbolt keepers shall be installed with long screws that attach securely to framing (new prehung doors equipped with reinforced metal plates on the back side of the jamb are also acceptable in lieu of long screws).

The finish of new hardware shall match existing hardware unless home owner selects otherwise.

##### Trade:	35	Wall Coverings/Ceilings/Trim

- 0244	Termite Damage

Inspect and open up the affected wall, beam, floor, and ceiling as needed to determine full extent of termite and moisture damage, including adjacent framing. Includes the wall between the den and kitchen.

Remove and replace all damaged structural members (studs, plates, beams, joists, subfloor) as necessary to restore full structural integrity, following current building codes and any required engineering direction.

Repair or replace insulation, wall and ceiling surfaces, trim, and flooring in the affected area, matching existing finishes as closely as reasonable.

Handle any necessary temporary adjustments to electrical, plumbing, or HVAC in the work area and restore them to code-compliant condition.

Obtain required permits and inspections, protect adjacent areas during construction, clean up on completion.

##### 0264   Ceiling Repair Throughout the House

Inspect all rooms for cracked, sagging, stained, or otherwise damaged drywall/plaster ceilings, including prior patch areas and visible water damage.

Remove loose, soft, or delaminated material in affected areas and open up as needed to confirm that underlying framing is sound and dry.

Replace damaged drywall/plaster with new materials of matching thickness; tape, mud, and sand to a smooth finish, including re-texturing to blend with existing surfaces as closely as practical.

Location:

3 - Interior

Approx. Wall SF: 0

Ceiling/Floor SF:	0

##### Spec #	Spec	Quantity	Units	Unit Price	Total Price

##### Trade:	35	Wall Coverings/Ceilings/Trim

Prime all repaired areas, and any stained surfaces, with an appropriate stain-blocking primer.

Apply finish paint to all repaired ceilings (and to logical break points such as entire ceilings as agreed), using

owner-approved colors and sheens, and feather into adjacent areas for a uniform appearance.

Repair or reinstall any trim, fixtures, or hardware disturbed during the work, perform paint touch-ups on affected trim, and leave rooms clean and ready for use.

| Trade: | 36 | Insulation |  |  |
| --- | --- | --- | --- | --- |
| 18 | 0266 | ATTIC INSULATION |  |  |
|  |  | Remove the homeowner's items stored in the attic, along with the existing attic insulation, and dispose of both properly. |  |  |
|  |  | Install a wooden or plywood dam around the access hatch high enough to hold back R-38 insulation. |  |  |
|  |  | Install continuous foam insulation around the perimeter of the panel where it rests on the casing. Glue an R-38 insulation batt cut to size on the top of the panel. |  |  |
|  |  | Install ventilation baffles per the Building Code. |  |  |
|  |  | Install insulation depth markers readily visible from the attic access hatch. |  |  |
|  |  | Blow in insulation to increase R value to R-38. |  |  |
|  |  | Attach insulation certificate to rafter at attic access, positioned to be easily seen. |  |  |
| Trade: | 38 | Stairs/Steps |  |  |
| 19 | 0286 | Folding Attic Stairs |  |  |
|  |  | Replace the utility folding stairway and install or adjust framing as needed to ensure proper installation in accordance with the manufacturer's specifications. Install casing that matches the existing trim in the room. Prime and paint the door and casing. Additionally, install a sealed, manufactured insulation cover over the attic stairs to achieve a minimum R-10 insulation value. |  |  |

#### Location Total:

#### Unit Total for 4418 Oak Drive, Unit Unit 01:

Address Grand Total for 4418 Oak Drive:

Bidder:

## Scope of Work for Lead Hazard Control

Address:	4418 Oak Dr	Date: 4/1/2026

All work described must be performed by a North Carolina state-certified lead firm, and a state lead permit is required. All work must comply with all applicable federal, state, and local regulations. Provide a separate price for each line item and add the total of line items at the end of the scope. Transfer the lead scope total to the lead control line item in the main scope of work.

| Lead Hazard (from LIRA) | Method of Control and Scope of Work | Number | Unit Cost | Line-Item Cost |
| --- | --- | --- | --- | --- |

| Side A through Side D – beige wood fascias, soffits, crown moldings and trim boards (Note: It appears some of the fascia boards have been randomly replaced.) | Remove and properly dispose of all gutters, downspouts, and crown molding. Remove all existing electrical, supports, and light fixtures, and store them safely for reinstallation. Install new white K-type .027 gauge seamless aluminum gutter to service the roof with sufficient pitch to new downspouts, install to construction manual specifications. Install 4” corrugated black pipe with the proper connectors to each downspout. Pipe length shall be long enough to ensure roof runoff flows away from the foundation. All damaged, loose, or rotted material associated with the required repair shall be removed and replaced. Cover fascia, soffits (including vents/install new where needed), trim boards, and drip boards with Tyvek and aluminum or vinyl. Reroute and support all existing cabling. Reinstall all light fixtures and ensure they are operational. |  |  |  |
| --- | --- | --- | --- | --- |

| Door A1 (to Room 1) – beige wood door casings, header, jambs and stops | Remove entire existing door assembly (including threshold); dispose of properly. Install a new Energy Star-rated pre-hung metal exterior door and threshold. Owner's choice of door style, color (door and threshold), and hardware finish. Insulate cavity. Install a single-cylinder deadbolt and lever-handled entry set door hardware, keyed alike. New casing and shoe molding will match the existing. Apply primer and topcoat. Inspecting from the crawlspace during the bid walk is recommended; this installation includes replacing all damaged, loose, or rotted material associated with the required repair. |  |  |  |
| --- | --- | --- | --- | --- |
| Room 4 – Door D1 – beige wood door casings, header, jambs and stops | Make smooth and operable, scrape loose paint, and re-paint. Owner's choice of color. |  |  |  |
| Room 1 – Window A1, Window A2 and Window A3 – beige vinyl mini-blinds (lead contained within the matrix of the vinyl) | Remove, properly dispose of, and replace with new. Owner's choice of color and style. |  |  |  |
| Room 2 – Window C1 – beige vinyl mini-blind (lead contained within the matrix of the vinyl) | Remove, properly dispose of, and replace with new. Owner's choice of color and style. |  |  |  |

| Room 3 – Closet D1 – beige vinyl vertical blind door (lead contained within the matrix of the vinyl) | Remove, properly dispose of, and replace with new. Owner's choice of color and style. |  |  |  |
| --- | --- | --- | --- | --- |
| Room 8 – Window A1 and Window D1 – beige vinyl miniblinds (lead contained within the matrix of the vinyl) | Remove, properly dispose of, and replace with new. Owner's choice of color and style. |  |  |  |
| Throughout – elevated lead dust levels | Complete specialized cleaning throughout the house. |  |  |  |

| All contractor requirements | The contractor is responsible for all project requirements, including but not limited to: All activities required by the City’s COVID-19 Precautions for Residential Housing Rehabilitation Activities policy. Obtaining all permits required. Said permits shall include all items in this scope of work. Provide temporary toilet facilities from the job start until the completion of work. Provide AS MANY roll-off dumpsters as needed without damaging the site. Collect construction debris using dust control methods. Remove dumpsters and repair any evidence of dumpster use. Contractor may haul debris away daily using dump trailers or trucks. |  |  |  |
| --- | --- | --- | --- | --- |

Total Bid for Lead Scope $

## Scope of Work for Asbestos Hazard Control

Address: 4418 Oak Dr	Date: 4/1/2026

All work disturbing asbestos containing materials (ACM) must be performed in compliance with all applicable federal, state, and local regulations. Work in the rehab scope that reasonably can be anticipated to disturb ACM is listed below. The contractor is responsible for identifying all activities that disturb ACM and for the proper removal and disposal of all ACM disturbed while working on this project.

If all disturbed ACM materials will fit into a single asbestos waste or glove bag, contractor's workers who are properly trained under OSHA Construction Standard Asbestos Regulations 29 CFR 1926.1101 may remove and dispose of the ACM using safe work practices.

If all disturbed ACM materials will not fit into a single asbestos waste or glove bag, or if workers are not properly trained, the contractor shall hire a state certified asbestos abatement firm to remove and dispose of the disturbed ACM materials.

Provide a separate price for each line item and add the total of line items at the end of the scope. Transfer the asbestos scope total to the asbestos control line item in the main scope of work.

| Asbestos Hazard (from asbestos report) | Work activities that will or may disturb identified Asbestos Containing Material (ACM) | Line Item Cost |
| --- | --- | --- |
| INTERIOR - Green Vinyl Flooring (UNDER Carpet) - Approximately 280 square feet of Green Vinyl Flooring (UNDER Carpet) on the Den floor (back left room). | Work that may disturb ACM includes, but is not limited to: *Cutting, drilling, sawing, sanding, or structurally altering in a way that the material could become friable. Air sampling clearance must be completed after disturbing ACM. |  |

Total Bid for Asbestos Scope: $