Invitation to Bid HNS 26-16

NOTE: Contractors are not authorized to visit the property before or after the bid

walk, without being accompanied by the City’s Rehabilitation Specialist.

Documents included in Package:

- Instructions to Bidders

- Specs by Location/Trade (Scope of Work)

- Subcontractor Certifications (if applicable)

- Floor Plan / Site Drawing (if applicable)

##### Bid Walk & Bid Opening:

| Project Address: SAFEHOME 1009 Rosada Dr Charlotte NC 28213 |  |
| --- | --- |
| Bid Walk: June 11th, 2026 @ 9:30 am |  |
| Bid Opening: June 18th, 2026 @ 5pm |  |
| Client Name: Dorris Massey |  |
| Project Manager: Stephon Blanding | Contact Number: 704-622-1685 |

Bid Walk and Bidding Instructions:

All bid walks are mandatory.

If you are going to be late the policy is the following:

Contact me BEFORE the start time if you are going to be late. If you are going to be more than 10 minutes late, we will proceed without you, and you will not be permitted to bid.

The day of a bid walk, the best way to reach me is 704-622-1685

Bids must be received by the date, time and place specified. All others will be considered nonresponsive and disqualified.

The Bids will be opened at 600 East Trade St. immediately following the above due date and time.

Company Acknowledgement: The undersigned, having become thoroughly familiar with the terms, conditions, limitations, and provisions of the housing improvement work to be performed at 1009 Rosada Dr Charlotte NC 28213 to be funded through the City of Charlotte Neighborhood & Business Services, in addition, having fully inspected the site in all particulars, hereby proposes and agrees to fully perform the work within the time stated and in strict accordance with the proposed contract documents including furnishing of any and all labor and materials, and to do all work required to complete said Work in accordance with the advised respective contractual, for the sum of money: All labor, materials, services and equipment necessary for the completion of the Work shown on the Drawings and in the Specifications: Dollars_($ )_ Written total

Specs Dated: 5/19/2026 Number of Pages: (10 pgs) 8 scope of work, 1 floor plan, 1 lead scope

Addenda # 1 Dated: Number of Pages:

Addenda # 2 Dated: Number of Pages:

Project Schedule: Minimum Start Date -

Completion Deadline:

Please Print and Sign:

Company Name/Firm:

Authorized Representative Name:

Signature: Date:

Requirements For Bidders

The City awards rehabilitation bids to the lowest responsive and responsible bidder. A responsible bidder for the safe home program is one who:

- Is a licensed general contractor in the State of North Carolina

- Has a Renovate, Repair & Paint Certification (http://www2.epa.gov/lead/renovation-repair-and-painting-program)

- Is not listed on a local, state or federal debarment list

- Carries an appropriate amount of insurance

- Can provide references verifying the contractor has completed work of a similar scope in a good workmanship like manner or successfully completed prior work for the Safe Home program. Referenced work must have been completed in one year or less from date of this invitation to bid.

A responsive bidder must:

- Submit all requested documentation on time

- Meet the above requirements for responsibility at the time of bid submittal

- Have the capacity to meet the required schedule for the project

- Existing rehab projects contracted by the contractor must be on schedule

The City reserves the right to waive any minor informalities or irregularities, which do not go to

the heart of the bid submittal or prejudice other offers, or to reject, for good and compelling reasons, any or all bid submittals.

Stephon Blanding

Rehabilitation Specialist City of Charlotte

Neighborhood and Business Services 600 E. Trade St.

Charlotte, NC 28202 PH:704-622-1685

email: stephon.blanding@charlottenc.gov

Instructions to Bidders

##### Explanations to Bidders

Any explanation desired by a Contractor regarding the meaning or interpretation of the advertisement for bids, drawings, specifications, etc., must be requested in writing to the Rehabilitation Specialist with sufficient time allowed for a reply to each Contractor before the submission of their bids. Any interpretation made will be in the form of an addendum to the invitation for bids, drawings, specifications, etc., and will be furnished to all prospective Contractors. The Contractor must acknowledge any revision to the bid documents in the space provided on the bid form and it must be submitted with their bid package.

##### Preparation of Bids

Bids shall be submitted on the forms furnished, or copies thereof, and must be manually signed. If erasures or other changes appear on the forms, the person signing the bids must initial each erasure or change

- The Contractor’s Bid Proposal must be properly executed and submitted on the form provided. Bids by Contractors must be received by the Rehabilitation Specialist at the time and place specified on the “Invitation to Bid”

- No bid will be considered unless all individual work items on the Bid Form including any addendums are priced. The sum of all work items must equal to the Total Bid Amount.

- Unless called for, alternate bids will not be considered

- Modification of bids already submitted will be considered if received at the office designated in the invitation for bids by the time set for opening of bids

##### Submittal of Bids

- Sealed bids will be submitted to the Safe Home Program Office as directed in the invitation to bid prior to or at the appointed bid opening time

- Bid will be time stamped on the date of delivery

- Each bid must be submitted in a sealed envelope bearing on the outside the name of the Contractor, the name of the project for which the bid is submitted, and the date and time of the bid opening

- All bids submitted must be typed or written in ink and signed by the Contractor's designated representative

- Failure to include all forms may result in rejection of a bid

- Required Bid Package Forms:

  - Scope of Work

  - Addenda Acknowledgement

  - Itemized Work Sheet

  - Subcontractor Certifications (if applicable)

##### Bid Validation

- All approved bids will be valid for 60 days after the bid opening date

HOUSING AND NEIGHBORHOOD SERVICES

http://housing.charlottenc.gov | 600 E. Trade Street | Charlotte, NC

City of Charlotte Housing and Neighborhood Services

600 E. Trade Street Charlotte, NC 28202

704-336-7600	5/20/2026

Location:	1 - Exterior	Approx. Wall SF: 0	Ceiling/Floor SF:	0

Spec #	Spec	Quantity	Units	Unit Price	Total Price

Trade:	4	Site Work

1	020

Regrade Foundation & Seed

Regrade yard to obtain proper drainage per code. Install 4" of top soil, seed, straw and fertilize disturbed area.

THE BACK YARD IS SLOPE TOWARDS THE HOUSE. THE GRADE NEED TO CHANGE. THIS WILL CORRECT THE FLOW OF WATER TO THE CRAWL SPACE. ON THE RT SIDE OF THE HOUSE, CREATE A SWELL TO DIVERT WATER AWAY FROM CRAWL AND TOWARDS THE STREET

1.00	EA

Address: 1009 Rosada Avenue	Unit:  Unit 01

| Trade: | 30 | Exterior Carpentry & Wall Framing |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | 048 | Soffit and Fascia Repair | 1.00 | EA |  |  |
|  |  | Replace deteriorated soffit and fascia areas, caulk, prep and Paint. Match existing material, trims and color. |  |  |  |  |
|  |  | THERE ARE SEVERAL AREAS ALONG THE FACIA AND SOFFIT THAT NEEDS REPAIR. FASTENERS ARE MISSING IN AREAS AND THE PRODUCTS IS NOT FUNCTIONING AS DESIGNED |  |  |  |  |
| Trade: | 32 | Roof/Chimney/Gutters/Downspouts |  |  |  |  |
| 3 | 4601 | **GUTTER & D.S./ACCESSORIES** | 1.00 | EA |  |  |
|  |  | THE DOWN SPOUTS ON THE RIGHT SIDE OF THE HOUSE NEED WATER DIVERTING SLEEVES ADDED TO THE DOWN SPOUT. THEY NEED TO EXTEND MINIMUM 2 FT AWAY FROM THE FOUNDATION AND INTO THE NEW SWELL |  |  |  |  |
| Trade: | 33 | Porch/Columns/Railings |  |  |  |  |
| 4 | 087 | Install Handrails | 1.00 | EA |  |  |

Location:	1 - Exterior	Approx. Wall SF: 0	Ceiling/Floor SF:	0

| Spec # | Spec | Quantity | Units | Unit Price | Total Price |
| --- | --- | --- | --- | --- | --- |
| Trade: 33 | Porch/Columns/Railings |  |  |  |  |
|  | INSTALL HANDRAILS AT THE BACK DOOR. INSTALL THEM ON BOTH SIDE OF THE STEPS. THEY SHOULD BE CONSTRUCTED WITH PT LUMBER. THE POST SHOULD BE 4X4 MATERIAL. THE SPINDLES OR RAILING SHALL BE CONSTRUCTED OR BUILT TO NC BUILDING CODE |  |  |  |  |
| 5 088 | Wrought Iron Railing (FRONT PORCH) | 1.00 | EA |  |  |
|  | Remove rust and deteriorated paint, prime and repaint wrought iron railings. |  |  |  |  |
|  | THE BASE OF THE RAILING NEED TO BE SECURED TO THE PORCH. THE EXISTING CONNECTOR IS NOT FUNCTIONING AS DESIGNED AND NEED TO BE REPLACED. |  |  |  |  |
|  | USE TAP CON SCREWS TO SECURE RAILING |  |  |  |  |

Location Total:

Location:	2 - Kitchen	Approx. Wall SF: 0	Ceiling/Floor SF:	0

| Spec # | Spec | Quantity | Units | Unit Price | Total Price |
| --- | --- | --- | --- | --- | --- |
| Trade: 20 | Flooring |  |  |  |  |
| 6 0228 | Resilient Flooring | 1.00 | EA |  |  |
|  | Install 25 year warrantied resilient floor covering per manufacturer's specifications. Flooring material is to be vinyl interlocking planks and be waterproof. Include transitions and painted or stained wood quarter-round at all perimeters to complete installation. |  |  |  |  |
| Trade: 34 | Doors & Windows |  |  |  |  |
| 7 0115 | Sliding Glass Patio Door | 1.00 | EA |  |  |
|  | the threshold just outside the door is not of proper thickness. There is a piece of aluminum there covering a piece of 3/4 ply. Remove existing material and correct the trip hazard. |  |  |  |  |
|  | MAKE SURE THE PLASTIC STRIP IS IN PLACE TO SEAL GAP BETWEEN SLIDING DOORS |  |  |  |  |
| Trade: 35 | Wall Coverings/Ceilings/Trim |  |  |  |  |
| 8 0250 | Prep & Paint Entire Interior | 1.00 | SF |  |  |
|  | Remove or cover hardware and accessories not to be painted. Scrape loose, peeling, cracked and blistered areas. Clean oil, grease, fungus, dirt and dust from surfaces. Fill holes and cracks. Prime all new materials and spot prime existing with acrylic latex. Top coat with owner's choice of low VOC acrylic FLAT latex in all habitable rooms and low VOC acrylic SEMI-GLOSS latex in kitchen and baths. Replace or uncover hardware, fixtures and accessories. Any moving of furniture |  |  |  |  |

Location:	2 - Kitchen	Approx. Wall SF: 0	Ceiling/Floor SF:	0

Spec #	Spec	Quantity	Units	Unit Price	Total Price

Trade:	35	Wall Coverings/Ceilings/Trim

required shall be included

THE ROOM IS BROWN PANELING.THE PANELING IS EXTENDED TO THE DINING ROOM. YOU WILL NEED TO PRIME THE PANEL FOR BEST PAINT ADHESION BEFORE PAINTING

| Trade: | 37 | Appliances-Cabinets-Accessories |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 9 | 0272 | Base Cabinets | 1.00 | EA |  |  |
|  |  | Replace base cabinets. Cabinets to be constructed of solid maple and plywood. No particle board allowed. See contractor's manual guidelines & specifications for full requirements. |  |  |  |  |
|  |  | ALL BASE CABINETS WILL BE REPLACED ALONG WITH THE UPPER. THE SINK, FAUCET, AND LINE CONNECTORS WILL ALSO BE REPLACED. |  |  |  |  |
|  |  | THE AREA BETWEEN THE DINING ROOM AND THE KITCHEN WILL BE CONVERTED TO AN ISLAND. THE OVERHANG WILL FACE THE DINING ROOM TO ACCOMODATES SEATING. |  |  |  |  |
| 10 | 0273 | Counter Tops Replace | 1.00 | LF |  |  |
|  |  | Screw to base cabinet a square edged plastic laminate countertop. Provide endcaps and cutout for sink. Caulk countertop to adjoining walls with low VOC caulking to match wall color. Owner's choice of in-stock color and texture. |  |  |  |  |
| 11 | 0274 | Wall Cabinets | 1.00 | EA |  |  |
|  |  | Replace wall cabinets. Cabinets are to be constructed of solid maple and plywood. No particle board allowed. See contractor's manual guidelines & specifications for full requirements. |  |  |  |  |
|  |  | THE CABINET ABOVE THE STOVE SHOULD BE SHORT ENOUGH TO ACCOMMODATE VENT HOOD BENEATH. THE VENT SHOULD BE HARD PIPED THRU THE ATTIC |  |  |  |  |

Location Total:

Location:	3 - Hall Bathroom	Approx. Wall SF: 0	Ceiling/Floor SF:	0

| Spec # | Spec | Quantity | Units | Unit Price | Total Price |
| --- | --- | --- | --- | --- | --- |
| Trade: 20 | Flooring |  |  |  |  |
| 12 0228 | Resilient Flooring | 1.00 | EA |  |  |
|  | Install 25 year warrantied resilient floor covering per manufacturer's specifications. Flooring material is to be vinyl interlocking planks and be waterproof. Include transitions and painted or stained wood quarter-round at all perimeters to complete installation. |  |  |  |  |

Location:	3 - Hall Bathroom	Approx. Wall SF: 0	Ceiling/Floor SF:	0

| Spec # | Spec | Quantity | Units | Unit Price | Total Price |
| --- | --- | --- | --- | --- | --- |
| Trade: 22 | Plumbing & Fixtures |  |  |  |  |
| 13 0224 | 17" Height Commode Replace | 1.00 | EA |  |  |
|  | Install a 17" height, 2 pieces, close coupled, white, vitreous China commode with a maximum water usage per flush of 1.6 gallons. Include plastic or pressed wood white seat, supply pipe, shut-off valve, flap valve and wax seal. |  |  |  |  |

Trade:	23	Electrical

| 14 | 8137 | UPDATE EXISTING ELECTRIC - BATHROOM 1.00 EA |
| --- | --- | --- |
|  |  | Update the electrical fixtures in the bathroom including: - One 20A GFCI receptacle located near sink with a 20 AMP circuit installed no more than 45" from the floor. - An ENERGY STAR approved ceiling mounted Fan/Light fixture, such as the NuTone QTREN080FLT,or a Broan QTXE080FLT capable of min. 80 CFM operating at 1 Sone or less, with an integral damper, and vented to the exterior. The fixture must accommodate 2 - GU24 fluorescent lamps. Switch fan & light using a single switch with a time delay for the fan such as the EFI Fan/Light Insulate the ductwork with vinyl or foil faced R 8 minimum duct insulation. Repair any damage to the ceiling installation and air seal fan/light assembly to the ceiling with low VOC caulk and air seal fan/light assembly to the ceiling with low VOC caulk. |
|  |  | ALSO INCLUDE A NEW LIGHT FIXTURE ABOVE THE VANITY |

Trade:	35	Wall Coverings/Ceilings/Trim

| 15 | 0250 | Prep & Paint Entire Interior | 1.00 | EA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Remove or cover hardware and accessories not to be painted. |  |  |  |  |
|  |  | Scrape loose, peeling, cracked and blistered areas. Clean oil, |  |  |  |  |
|  |  | grease, fungus, dirt and dust from surfaces. Fill holes and |  |  |  |  |
|  |  | cracks. Prime all new materials and spot prime existing with |  |  |  |  |
|  |  | acrylic latex. Topcoat with owner's choice of low VOC acrylic |  |  |  |  |
|  |  | FLAT latex in all habitable rooms and low VOC acrylic |  |  |  |  |
|  |  | SEMI-GLOSS latex in kitchen and baths. Replace or uncover |  |  |  |  |
|  |  | hardware, fixtures and accessories. Any moving of furniture |  |  |  |  |
|  |  | required shall be included |  |  |  |  |

| Trade: | 37 | Appliances-Cabinets-Accessories |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 16 | 0281 | 24" Towel Bar | 2.00 | EA |  |  |
|  |  | Install a new wall mounted 24" towel bar. |  |  |  |  |
| 17 | 0284 | Vanity/ Countertop/ Sink | 1.00 | EA |  |  |
|  |  | Install new vanity cabinet complete with countertop of laminate with vitreous china sink or marble-lite top with preformed sink. Include single handled metal faucet with drain and pop-up, P-trap, supply lines, full port ball type shut-off valves & escutcheon plates. Vanity medicine cabinet/mirror to remain. |  |  |  |  |

Location Total:

Location:	5 - Hall	Approx. Wall SF: 0	Ceiling/Floor SF:	0

| Spec # | Spec | Quantity | Units | Unit Price | Total Price |
| --- | --- | --- | --- | --- | --- |
| Trade: 20 | Flooring |  |  |  |  |
| 18 0228 | Resilient Flooring | 1.00 | EA |  |  |
|  | Install 25 year warrantied resilient floor covering per manufacturer's specifications. Flooring material is to be vinyl interlocking planks and be waterproof. Include transitions and painted or stained wood quarter-round at all perimeters to complete installation. |  |  |  |  |
|  | REMOVE AND DISPOSE EXISTING CARPET FROM DINING ROOM TO THE END OF THE HALLWAY INCLUDE COST OF SHOE MOLDING INSTALLED, CAULKED AND PAINTED |  |  |  |  |

Trade:	34	Doors & Windows

19	0111

Slab Door Interior

Install a slab door to existing jam, mortise in hinges and latch. Hole saw for the hardware. Include new hardware. Locking for bedrooms and bathroom, passage for all others. Match existing style and finish.

7.00	EA

Location Total:

Location:	6 - Bedroom	Approx. Wall SF: 0	Ceiling/Floor SF:	0

Spec #	Spec	Quantity	Units	Unit Price	Total Price

Trade:	23	Electrical

| 20 | 0183 | Smoke Detector Hard Wired | 3.00 | EA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Install UL approved, interconnected, ceiling mounted smoke and heat detectors permanently wired into a receptacle box with battery backups in bedrooms and outside of sleeping areas. |  |  |  |  |

Location Total:

Location:	7 - Bedroom Back Rt	Approx. Wall SF: 0	Ceiling/Floor SF:	0

| Spec # | Spec | Quantity | Units | Unit Price | Total Price |
| --- | --- | --- | --- | --- | --- |
| Trade: 23 | Electrical |  |  |  |  |
| 21 0190 | Ceiling Fan with Light Kit | 1.00 | EA |  |  |
|  | Install a 52" 3 speed flush mounted ceiling fan with a minimum two bulb UL approved LED light fixture with shade and lamps. Include ceiling fan mounting box. New fan is to be Energy Star rated. |  |  |  |  |
|  | MAKE SURE THE FAN HAS A REMOTE |  |  |  |  |

Location Total:

Location:	8 - Rear left master Bathroom	Approx. Wall SF: 0	Ceiling/Floor SF:	0

Spec #	Spec	Quantity	Units	Unit Price	Total Price

Trade:	23	Electrical

| 22 | 7575 | RECEPTACLE--20 AMP CIRCUIT w/ Light switch | 1.00 | EA |
| --- | --- | --- | --- | --- |
|  |  | Install 20-amp, ivory, duplex receptacle with a matching plastic cover plate on a separate circuit with an individual over protection device. Fish wire and repair all tear out. If installed over a countertop install no more than 46 inches above floor height. If installed as a standard wall receptacle install at least 15 inches above the floor. |  |  |
|  |  | INSTALL 20 AMP GFI SOCKET TO THE RT OF THE VANITY. IN THE DUPLEX BOX INSTALL A SINGLE POLE LIGHT SWITCH FOR NEW LIGHT FIXTURE. INCLUDE THE LIGHT FIXTURE IN THE PRICING |  |  |

Location Total:

Location:	9 - Middle Bedroom	Approx. Wall SF: 0	Ceiling/Floor SF:	0

Spec #	Spec	Quantity	Units	Unit Price	Total Price

Trade:	23	Electrical

23	0192

LIGHT FIXTURE CEILING

Replace existing ceiling mounted fixture with 2 bulb UL approved LED light fixture with shade and lamps.

1.00	EA

| Trade: | 35 | Wall Coverings/Ceilings/Trim |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 24 | 0243 | Baseboard Install | 1.00 | EA |  |  |
|  |  | Install wooden baseboards free of surface defects, fastened with finished nails, properly set and puttied. Repair work should match existing. |  |  |  |  |
|  |  | THE BASEBOARD ON WALL C IS MISSING. INSTALL NEW PIECE AND PAINT TO MATCH THE EXISTING (WHITE) |  |  |  |  |
| 25 | 0251 | PAINT ENTIRE ROOM | 1.00 | DY |  |  |
|  |  | Remove or cover hardware, fixtures, furniture, accessories, and other room contents that are not to be painted. |  |  |  |  |
|  |  | Scrape loose, peeling, cracked and blistered areas. Clean oil, grease, fungus, dirt, and dust from surfaces. Fill holes and cracks. Repair any remaining defects in surfaces as necessary to produce smooth paint ready surfaces. |  |  |  |  |
|  |  | Prime all materials with a stain killing primer. NOTE: Very old paint or other site conditions may require the use of a bonding primer to achieve a durable finished product. The contractor is responsible for choosing products and processes that ensure a durable finished product. |  |  |  |  |
|  |  | Topcoat with two coats of low VOC acrylic latex paint. Use semi-gloss paint in kitchens and bathrooms. Use flat or eggshell sheens (owner’s choice) in all other rooms. |  |  |  |  |

Location:	9 - Middle Bedroom	Approx. Wall SF: 0	Ceiling/Floor SF:	0

Spec #	Spec	Quantity	Units	Unit Price	Total Price

Trade:	35	Wall Coverings/Ceilings/Trim

Surfaces to be painted include all walls, ceiling, trim, and doors. Interiors of closet are not included unless so specified.

Ceilings shall be painted with the owner’s choice of ceiling white or the selected wall color.

Replace or uncover hardware, fixtures, furniture, accessories, and other room contents.

Location Total:

Location:	10 - Foundation/Crawl Space	Approx. Wall SF: 0	Ceiling/Floor SF:	0

Spec #	Spec	Quantity	Units	Unit Price	Total Price

Trade:	20	Flooring

| 26 | 0240 | Drop Girder and Masonry Piers 1.00 AL |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Jack sagging floor joists up with a temp beam and bottle jacks. Build piers and girders. Dig ground 18"x18" for footing 12" below grade. Pour 2500 psi concrete footing 10" thick. Build piers by mortaring 8"x16" concrete blocks, with a pressure treated block of lumber on top. Build seamless girder with dimensional lumber, sized by the span chart per the NC Building Code. After new masonry piers have dried and cured, remove temp beam and bottle jacks. |  |  |  |  |
|  |  | THE BACK LEFT BEDROOM FLOOR HAS SOME DEFLECTION. TO CONTROL THE DEFLECTION, PLACE A DROP GIRDER AT THE MIDDLE SPAN OF THE EXISTING FLOOR JOIST. MAKE SURE THE GIRDER RUNS PARALELL TO THE MAIN HOUSE EXISTING GIRDER. THE PIERS SHOULD BE PLACED 5FT N CENTER. THE DROP GIRDER CAN BE PRESSURE TREATED 4X6 |  |  |  |  |
| Trade: | 29 | Foundation & Crawl Space |  |  |  |  |
| 27 | 038 | FOUNDATION AUTOMATIC | 14.00 | EA |  |  |
|  |  | Install new automatic foundation vents at existing vent openings. |  |  |  |  |
|  |  | Owner’s choice of black or brown. |  |  |  |  |

Location Total:

Location:	11- Living Room	Approx. Wall SF: 0	Ceiling/Floor SF:	0

Spec #	Spec	Quantity	Units	Unit Price	Total Price

Trade:	10	Carpentry

28	3080

THRESHOLD-METAL

1.00	EA

Location:	11- Living Room	Approx. Wall SF: 0	Ceiling/Floor SF:	0

Spec #	Spec	Quantity	Units	Unit Price	Total Price

Trade:	10	Carpentry

THE THRESHOLD AT THE FRONT DOOR NEEDS REPAIR

| Trade: | 34 | Doors & Windows |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 29 | 094 | WINDOW REPAIR LIVING RM | 1.00 | EA |  |  |
|  |  | Refurbish existing wood windows to function as intended. provide an air tight seal when closed, and lock securely. |  |  |  |  |
|  |  | Scope includes: |  |  |  |  |
|  |  | THE 2 CENTER WINDOWS IN THE BAY WINDOW HAS A BROKEN SEAL. THE WINDOWS ARE LOOKING VERY CLOUDY. PLEASE REPLACE WITH LOW E WINDOW. YOU ARE REPLACING JUST THE PANES |  |  |  |  |

Location Total:

Unit Total for 1009 Rosada Avenue, Unit Unit 01:		 Address Grand Total for 1009 Rosada Avenue:

Bidder:

## Scope of Work for Lead Hazard

Address: 1009 Rosada Dr	Date: 5/27/2026

All work described must be performed by a North Carolina state certified lead firm and a state lead permit is required. All work must comply with all applicable federal, state, and local regulations. Provide a separate price for each line item and add the total of line items at the end of the scope. Transfer the lead scope total to the lead control line item in the main scope of work.

| Lead Hazard (from LIRA) | Method of Control and Scope of Work | Number | Unit Cost | Line Item Cost |
| --- | --- | --- | --- | --- |
| Contractor requirement | The contractor is responsible for all project requirements, including but not limited to: Obtaining all permits required. Said permits shall include all items in this scope of work. Provide temporary toilet facilities from job start until the completion of work. Provide AS MANY roll-off dumpsters as needed without damaging the site. Collect construction debris using dust control methods. Remove dumpsters and repair any evidence of the dumpster’s use. Contractors may haul debris away daily using dump trailers or trucks. |  |  |  |
| Room 1 - Floor – elevated lead dust level on floor | Complete specialized cleaning throughout Room 1. |  |  |  |

Total Bid for Lead Scope $

1