# Invitation to Bid HNS 26-12

NOTE: Contractors are not authorized to visit the property before or after the bid

walk, without being accompanied by the City’s Rehabilitation Specialist.

Documents included in Package:

- Instruction to Bidders

- Specs by Location/Trade (Scope of Work)

- Subcontractor Certifications (if applicable)

- Floor Plan / Site Drawing (if applicable)

### Bid Walk & Bid Opening:

| Project Address: 329 North Hoskins Road, Charlotte, NC 28216 |  |
| --- | --- |
| Bid Walk: 5/27/26 @ 10am |  |
| Bid Opening: 6/3/26 @ 2pm |  |
| Client Name: |  |
| Project Manager: Daniel Edwards | Contact Number: 980-214-4133 |

Bid Walk and Bidding Instructions:

All bid walks are mandatory.

If you are going to be late the policy is the following:

Contact me BEFORE the start time if you are going to be late. If you are going to be more than 10 minutes late, we will proceed without you and you will not be permitted to bid.

The day of a bid walk the best way to reach me is at _980-214-4133_

Bids must be received by the date, time and place specified. All others will be considered non responsive and disqualified.

## The Bids will be opened at 600 East Trade St. immediately following the above due date and time.

Company Acknowledgement: The undersigned, having become thoroughly familiar with the terms, conditions, limitations, and provisions of the housing improvement work to be performed at ______________________ to be funded through the City of Charlotte Neighborhood & Business Services, in addition, having fully inspected the site in all particulars, hereby proposes and agrees to fully perform the work within the time stated and in strict accordance with the proposed contract documents including furnishing of any and all labor and materials, and to do all work required to complete said Work in accordance with the advised respective contractual, for the sum of money: All labor, materials, services and equipment necessary for the completion of the Work shown on the Drawings and in the Specifications: Dollars_($ )_ Written total

Specs Dated: Number of Pages:

Addenda # 1 Dated: Number of Pages:

Addenda # 2 Dated: Number of Pages:

Project Schedule: Minimum Start Date -

Completion Deadline:

Please Print and Sign:

Company Name/Firm:

Authorized Representative Name:

Signature: Date:

Requirements For Bidders

The City awards rehabilitation bids to the lowest responsive and responsible bidder. A responsible bidder for the safe home program is one who:

- Is a licensed general contractor in the State of North Carolina

- Has a Renovate, Repair & Paint Certification (http://www2.epa.gov/lead/renovation-repair-and-painting-program)

- Is not listed on a local, state or federal debarment list

- Carries an appropriate amount of insurance

- Can provide references verifying the contractor has completed work of a similar scope in a good workmanship like manner or successfully completed prior work for the Safe Home program. Referenced work must have been completed in one year or less from date of this invitation to bid.

A responsive bidder must:

- Submit all requested documentation on time

- Meet the above requirements for responsibility at the time of bid submittal

- Have the capacity to meet the required schedule for the project

- Existing rehab projects contracted by the contractor must be on schedule

The City reserves the right to waive any minor informalities or irregularities, which do not go to

the heart of the bid submittal or prejudice other offers, or to reject, for good and compelling reasons, any or all bid submittals.

Rehabilitation Specialist City of Charlotte

Neighborhood and Business Services 600 E. Trade St.

Charlotte, NC 28202 PH:

Fax:

Instructions to Bidders

### Explanations to Bidders

Any explanation desired by a Contractor regarding the meaning or interpretation of the advertisement for bids, drawings, specifications, etc., must be requested in writing to the Rehabilitation Specialist with sufficient time allowed for a reply to each Contractor before the submission of their bids. Any interpretation made will be in the form of an addendum to the invitation for bids, drawings, specifications, etc., and will be furnished to all prospective Contractors. The Contractor must acknowledge any revision to the bid documents in the space provided on the bid form and it must be submitted with their bid package.

### Preparation of Bids

Bids shall be submitted on the forms furnished, or copies thereof, and must be manually signed. If erasures or other changes appear on the forms, the person signing the bids must initial each erasure or change

- The Contractor’s Bid Proposal must be properly executed and submitted on the form provided. Bids by Contractors must be received by the Rehabilitation Specialist at the time and place specified on the “Invitation to Bid”

- No bid will be considered unless all individual work items on the Bid Form including any addendums are priced. The sum of all work items must equal to the Total Bid Amount.

- Unless called for, alternate bids will not be considered

- Modification of bids already submitted will be considered if received at the office designated in the invitation for bids by the time set for opening of bids

### Submittal of Bids

- Sealed bids will be submitted to the Safe Home Program Office as directed in the invitation to bid prior to or at the appointed bid opening time

- Bid will be time stamped on the date of delivery

- Each bid must be submitted in a sealed envelope bearing on the outside the name of the Contractor, the name of the project for which the bid is submitted, and the date and time of the bid opening

- All bids submitted must be typed or written in ink and signed by the Contractor's designated representative

- Failure to include all forms may result in rejection of a bid

- Required Bid Package Forms:

  - Scope of Work

  - Addenda Acknowledgement

  - Itemized Work Sheet

  - Subcontractor Certifications (if applicable)

### Bid Validation

- All approved bids will be valid for 60 days after the bid opening date

HOUSING AND NEIGHBORHOOD SERVICES

http://housing.charlottenc.gov | 600 E. Trade Street | Charlotte, NC

City of Charlotte Housing and Neighborhood Services

600 E. Trade Street Charlotte, NC 28202

704-336-7600	4/10/2026

| Location: | 1 - HVAC Approx. Wall SF: 0 | Ceiling/Floor SF: | 0 |
| --- | --- | --- | --- |
| Spec # | Spec Quantity | Units Unit Price | Total Price |
| Trade: 21 | HVAC |  |  |
| 1 0144 | Gas Pack 1.00 | AL |  |
|  | Install combination AC/furnace unit, minimum 14 SEER design per Manual J. |  |  |

Location Total:

Location:	2 - Interior	Approx. Wall SF: 0	Ceiling/Floor SF:	0

Spec #	Spec	Quantity	Units	Unit Price	Total Price

Trade:	34	Doors & Windows

2	099

3	0115

Vinyl Window

Dispose of window units and any security bars. Field measure, order and install new Energy Star rated vinyl, double glazed LOW- E windows including half screens. Insulate rough opening. Install new extension jambs, casing and sill as needed, prime and paint. Exterior trim to be wrapped in PVC coated aluminum coil as needed. Repair all areas disturbed by removal and installation. This installation is to include repairing any damaged header, framing (to include king studs, jack studs etc.) sub-floor, floor joists, band joists and sill. Any windows replaced in a bathroom must have tempered glass.

Sliding Glass Patio Door

Remove and dispose of existing door, frame and threshold. Install new Energy Star rated pre-hung exterior vinyl siding patio door with screen and locking hardware. Insulate cavity. New casing and shoe molding will match existing. This installation is to include repairing any damaged header, door framing, wall framing, sub-floor, floor joists, band joists, and sill.

15.00	EA

1.00	EA

Address: 329 N Hoskins Road	Unit:  Unit 01

Location Total:

Location:	3 - Interior	Approx. Wall SF: 0	Ceiling/Floor SF:	0

Spec #	Spec	Quantity	Units	Unit Price	Total Price

| Trade: | 23 | Electrical |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 4 | 0147 | 200 AMP ELECTRICAL SERVICE | 1.00 | AL |  |  |
|  |  | Replace electrical service with new 110/220 volt, 200 amp panel with main disconnect and 24 circuit panel board; meter socket; weather head; service cable; and ground rods and cable. Caulk exterior service penetrations. Label circuits legibly. |  |  |  |  |
|  |  | Replacement of all sub-panels is included. |  |  |  |  |
|  |  | Install GFCI breakers on all two wire receptacle circuits. Install GFCI/AFCI breakers on all bedroom receptacle circuits. Label outlets per Code requirements. |  |  |  |  |
| 5 | 0148 | Light Fixture Replace | 2.00 | EA |  |  |
|  |  | Replace or install a ceiling mounted 2 bulb or wall mounted 4 bulb strip, UL approved, LED light fixture with shade and lamps. |  |  |  |  |
| 6 | 0172 | Terminate Wire in Box | 1.00 | EA |  |  |
|  |  | Terminate existing wire in a flush mounted, Code approved box. |  |  |  |  |
| 7 | 0180 | Combination CO / Smoke Detector Hard Wired | 4.00 | EA |  |  |
|  |  | Install a hard wired combination carbon monoxide and smoke detector with battery back up. |  |  |  |  |
| 8 | 0190 | Ceiling Fan with Light Kit | 3.00 | EA |  |  |
|  |  | Install a 52" 3 speed flush mounted ceiling fan with a minimum two bulb UL approved, LED light fixture with shade and lamps. |  |  |  |  |

Include ceiling fan mounting box. New fan is to be Energy Star rated.

Location Total:

Location:	4 - Roof	Approx. Wall SF: 0	Ceiling/Floor SF:  0

Spec #	Spec	Quantity	Units	Unit Price	Total Price

Trade:	32	Roof/Chimney/Gutters/Downspouts

9	067

Tear Off & Reroof Shingles

Remove and dispose of existing roofing, inspect and repair roof deck (include 96 sq.ft. of decking replacement in bid) and install new 30 year architectural shingles. Include ice and water shield at all roof edges, felt paper, shingle over ridge vent, boots, flashing and all necessary accessories.

1.00	AL

Location Total:

Location:	5 - Kitchen	Approx. Wall SF: 0	Ceiling/Floor SF:  0

Spec #	Spec	Quantity	Units	Unit Price	Total Price

Location:	5 - Kitchen	Approx. Wall SF: 0	Ceiling/Floor SF:	0

Spec #	Spec	Quantity	Units	Unit Price	Total Price

Trade:	25	Appliances

| 10 | Custom | Replace Gas Cooktop with Electric Range | 1.00 | AL |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Remove and dispose of the existing gas cooktop. Cap the gas |  |  |  |  |
|  |  | line per local code. Install a new 240V dedicated electrical |  |  |  |  |
|  |  | circuit and outlet for a new electric range. Modify cabinetry and |  |  |  |  |
|  |  | countertops to accommodate the new appliance footprint. |  |  |  |  |
|  |  | Perform final installation and leveling. |  |  |  |  |

| Trade: | 37 | Appliances-Cabinets-Accessories |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 11 | 0272 | Base Cabinets | 1.00 | AL |  |  |
|  |  | Replace base cabinets. Cabinets to be constructed of solid maple and plywood. No particle board allowed. See contractor's manual guidelines & specifications for full requirements. |  |  |  |  |
| 12 | 0278 | KITCHEN COUNTERTOPS | 1.00 | AL |  |  |
|  |  | Remove existing counter tops and dispose of properly. |  |  |  |  |
|  |  | Measure cabinets for new tops and order built to size post formed laminate tops from a reputable top shop. Top shop shall apply all end caps and supply side splashes where cabinets end at walls. Top shop shall perform all sink cut outs. |  |  |  |  |
|  |  | NOTE: Do not buy counter top blanks at big box stores and cut them to size on site. |  |  |  |  |
|  |  | Where counter top sections join, such as corners, caulk joints and install connectors supplied by the top shop to join the tops together, taking care to maintain a tight and flush transition all along the joint. |  |  |  |  |
|  |  | Attach counter tops to cabinets with screws front and back at all top ends, both sides of sinks, and at least every 3’ in between. |  |  |  |  |
|  |  | If counter tops do not fit tightly to wall, trim drywall and/or backsplash margin as necessary to maintain a caulkable gap. Neatly caulk counter tops to adjoining walls with low VOC matching caulk. |  |  |  |  |
|  |  | Provide owner with a minimum of five pattern choices that coordinate well with the cabinets. |  |  |  |  |
| 13 | Custom | Remove Wall Oven and Replace with Cabinet. | 1.00 | AL |  |  |
|  |  | Disconnect and remove the existing wall oven. Properly terminate electrical wiring in accordance with local building codes. Modify the oven cabinet or install a new cabinet insert to match the existing kitchen layout, including all necessary trim and hardware |  |  |  |  |

Location Total:

Location:	6 - Exterior	Approx. Wall SF: 0	Ceiling/Floor SF:	0

Location:	6 - Exterior	Approx. Wall SF: 0	Ceiling/Floor SF:	0

| Spec # | Spec | Quantity | Units | Unit Price | Total Price |
| --- | --- | --- | --- | --- | --- |
| Trade: 4 | Site Work |  |  |  |  |
| 14 021 | PEST CONTROL | 1.00 | SF |  |  |
| Provide owner with a one year service contract with a properly licensed exterminator. |  |  |  |  |  |
| Service shall include initial treatment for all observed insect infestations with quarterly follow up treatments over the course of the contract. |  |  |  |  |  |
| Contract shall also provide for callback treatments between scheduled treatments if an infestation re-appears. |  |  |  |  |  |
| NOTE: Contractor shall inform exterminator that automatic contract renewals are not permitted. Contracts may only be renewed with the express, informed consent of the home owner. |  |  |  |  |  |

Location Total:

Unit Total for 329 N Hoskins Road, Unit Unit 01:		 Address Grand Total for 329 N Hoskins Road:

Bidder:

# Scope of Work for Lead Hazard Control

## 329 N. Hoskins Road	Date 4/9/26

All work described must be performed by a North Carolina state certified lead firm and a state lead permit is required. All work must comply with all applicable federal, state, and local regulations. Provide a separate price for each line item and add the total of line items at the end of the scope. Transfer the lead scope total to the lead control line item in the main scope of work.

|  | Method of Control and Scope of Work | Number | Unit Cost | Line Item Cost |
| --- | --- | --- | --- | --- |
| Exterior |  |  |  |  |
|  | The contractor is responsible for all project requirements, including but not limited to: Obtaining all permits required. Said permits shall include all items in this scope of work. Provide temporary toilet facilities from job start until the completion of work. Provide AS MANY roll-off dumpsters as needed without damaging the site. Collect construction debris using dust control methods. Remove dumpsters and repair any evidence of the dumpster use. Contractor may haul debris away daily using dump trailers or trucks. |  |  |  |
| Side A through Side D – white wood fascias, soffits and trim boards and red wood crown moldings | Cover with Tyvek and aluminum or vinyl. |  |  |  |

| Side A through Side D – white wood and metal window and door lintels | Cover with Tyvek and aluminum or vinyl. |  |  |  |
| --- | --- | --- | --- | --- |
| Side C Carport– white wood wall, ceiling, crown moldings, ceiling support beams, trim boards and window casings, headers and sills for brick privacy windows. | Cover with Tyvek and aluminum or vinyl. |  |  |  |
| Closet Door C1 – white wood door, casings, header, jambs and stops | Replace door, jambs and stops and cover door casings and header with Tyvek and aluminum. |  |  |  |
| INTERIOR |  |  |  |  |
| Room 1 – Living Room – Window A4 – beige vinyl mini-blind (lead contained within the matrix of the vinyl) | Remove and complete specialized cleaning. |  |  |  |
| Room 4 – Bath 2 – Window C1 – beige vinyl mini-blind (lead contained within the matrix of the vinyl) | Remove and complete specialized cleaning. |  |  |  |
| Room 2 – elevated lead dust level on floor | Complete specialized cleaning throughout house. |  |  |  |
| Throughout – elevated lead dust levels | Complete specialized cleaning of Room 2 including Door B1 door and components. |  |  |  |

Total Bid for Lead Scope $