# Invitation to Bid HNS 26-18

NOTE: Contractors are not authorized to visit the property before or after the bid

walk, without being accompanied by the City’s Rehabilitation Specialist.

Documents included in Package:

- Instruction to Bidders

- Specs by Location/Trade (Scope of Work)

- Subcontractor Certifications (if applicable)

- Floor Plan / Site Drawing (if applicable)

Bid Walk & Bid Opening:

| Project Address: 4401 Gatewood Drive |  |
| --- | --- |
| Bid Walk: 7/21/26 @ 10am |  |
| Bid Opening: 7/28/26 @ 2pm |  |
| Client Name: |  |
| Project Manager: Daniel Edwards | Contact Number: 980-214-4133 |

Bid Walk and Bidding Instructions:

All bid walks are mandatory.

If you are going to be late the policy is the following:

Contact me BEFORE the start time if you are going to be late. If you are going to be more than 10 minutes late, we will proceed without you and you will not be permitted to bid.

The day of a bid walk the best way to reach me is at _980-214-4133

Bids must be received by the date, time and place specified. All others will be considered non responsive and disqualified.

# The Bids will be opened at 600 East Trade St. immediately following the above due date and time.

Company Acknowledgement: The undersigned, having become thoroughly familiar with the terms, conditions, limitations, and provisions of the housing improvement work to be performed at ______________________ to be funded through the City of Charlotte Neighborhood & Business Services, in addition, having fully inspected the site in all particulars, hereby proposes and agrees to fully perform the work within the time stated and in strict accordance with the proposed contract documents including furnishing of any and all labor and materials, and to do all work required to complete said Work in accordance with the advised respective contractual, for the sum of money: All labor, materials, services and equipment necessary for the completion of the Work shown on the Drawings and in the Specifications: Dollars_($ )_ Written total

Specs Dated: Number of Pages:

Addenda # 1 Dated: Number of Pages:

Addenda # 2 Dated: Number of Pages:

Project Schedule: Minimum Start Date -

Completion Deadline:

Please Print and Sign:

Company Name/Firm:

Authorized Representative Name:

Signature: Date:

Requirements For Bidders

The City awards rehabilitation bids to the lowest responsive and responsible bidder. A responsible bidder for the safe home program is one who:

- Is a licensed general contractor in the State of North Carolina

- Has a Renovate, Repair & Paint Certification (http://www2.epa.gov/lead/renovation-repair-and-painting-program)

- Is not listed on a local, state or federal debarment list

- Carries an appropriate amount of insurance

- Can provide references verifying the contractor has completed work of a similar scope in a good workmanship like manner or successfully completed prior work for the Safe Home program. Referenced work must have been completed in one year or less from date of this invitation to bid.

A responsive bidder must:

- Submit all requested documentation on time

- Meet the above requirements for responsibility at the time of bid submittal

- Have the capacity to meet the required schedule for the project

- Existing rehab projects contracted by the contractor must be on schedule

The City reserves the right to waive any minor informalities or irregularities, which do not go to

the heart of the bid submittal or prejudice other offers, or to reject, for good and compelling reasons, any or all bid submittals.

Rehabilitation Specialist City of Charlotte

Neighborhood and Business Services 600 E. Trade St.

Charlotte, NC 28202 PH:

Fax:

Instructions to Bidders

Explanations to Bidders

Any explanation desired by a Contractor regarding the meaning or interpretation of the advertisement for bids, drawings, specifications, etc., must be requested in writing to the Rehabilitation Specialist with sufficient time allowed for a reply to each Contractor before the submission of their bids. Any interpretation made will be in the form of an addendum to the invitation for bids, drawings, specifications, etc., and will be furnished to all prospective Contractors. The Contractor must acknowledge any revision to the bid documents in the space provided on the bid form and it must be submitted with their bid package.

Preparation of Bids

Bids shall be submitted on the forms furnished, or copies thereof, and must be manually signed. If erasures or other changes appear on the forms, the person signing the bids must initial each erasure or change

- The Contractor’s Bid Proposal must be properly executed and submitted on the form provided. Bids by Contractors must be received by the Rehabilitation Specialist at the time and place specified on the “Invitation to Bid”

- No bid will be considered unless all individual work items on the Bid Form including any addendums are priced. The sum of all work items must equal to the Total Bid Amount.

- Unless called for, alternate bids will not be considered

- Modification of bids already submitted will be considered if received at the office designated in the invitation for bids by the time set for opening of bids

Submittal of Bids

- Sealed bids will be submitted to the Safe Home Program Office as directed in the invitation to bid prior to or at the appointed bid opening time

- Bid will be time stamped on the date of delivery

- Each bid must be submitted in a sealed envelope bearing on the outside the name of the Contractor, the name of the project for which the bid is submitted, and the date and time of the bid opening

- All bids submitted must be typed or written in ink and signed by the Contractor's designated representative

- Failure to include all forms may result in rejection of a bid

- Required Bid Package Forms:

  - Scope of Work

  - Addenda Acknowledgement

  - Itemized Work Sheet

  - Subcontractor Certifications (if applicable)

Bid Validation

- All approved bids will be valid for 60 days after the bid opening date

HOUSING AND NEIGHBORHOOD SERVICES

http://housing.charlottenc.gov | 600 E. Trade Street | Charlotte, NC

City of Charlotte Housing and Neighborhood Services

600 E. Trade Street Charlotte, NC 28202

704-336-7600	6/2/2026

Location:	1 - Kitchen	Approx. Wall SF: 0	Ceiling/Floor SF:	0

Spec #	Spec	Quantity	Units	Unit Price	Total Price

Trade:	22	Plumbing & Fixtures

1	6830

SINK-SINGLE BOWL COMPLETE

Install a 22-gauge, 25"x22"x7" deep, single bowl, stainless steel, self rimming kitchen sink including a steel, metal body faucet, rated at 2.0 GPM or less, with a 15-year drip-free warranty, grease trap, supply lines, full port ball type shut-off valves & escutcheon plates on all supply & drain lines. NOTE: All copper is to be soldered (no compression fittings) & all PVC fittings except for the trap glued.

1.00	EA

Location Total:

Location:	2 - Water Heater	Approx. Wall SF: 0	Ceiling/Floor SF:	0

Spec #	Spec	Quantity	Units	Unit Price	Total Price

Trade:	22	Plumbing & Fixtures

2	0220

Water Heater 40 Gallon Electric

Install a 40-gallon, glass lined, high recovery insulated to R-7, double element, electric water heater with a 6-year warranty. Include pressure and temperature relief valve, expansion tank, discharge tube to within 6" of floor or to outside of structure, shut-off valve and electric supply. If it is located on an upper floor or if there is no floor drain, install a catch pan drained to the exterior. Dispose of old water heater appropriately.

1.00	EA

Address: 4401 Gatewood Drive	Unit:  Unit 01

Location Total:

Location:	3 - Smoke/CO	Approx. Wall SF: 0	Ceiling/Floor SF:	0

Spec #	Spec	Quantity	Units	Unit Price	Total Price

Location:	3 - Smoke/CO	Approx. Wall SF: 0	Ceiling/Floor SF:	0

| Spec # | Spec | Quantity | Units | Unit Price | Total Price |
| --- | --- | --- | --- | --- | --- |
| Trade: 23 | Electrical |  |  |  |  |
| 3 0170 | SMOKE AND CARBON MONOXIDE DETECTORS | 1.00 | EA |  |  |
|  | Install UL approved ceiling mounted smoke and heat detectors permanently hard wired into outlet boxes with battery backups in all bedrooms and outside of all sleeping areas. |  |  |  |  |
|  | Detector in the hallway shall be a combination CO/smoke detector. |  |  |  |  |
|  | All detectors shall be interconnected so that when any one detector goes off, all other detectors also go off. |  |  |  |  |
|  | Installation shall comply with all requirements of the Electrical Code SMOKE AND CARBON MONOXIDE DETECTORS |  |  |  |  |

Location Total:

Location:	4 - Lead Scope	Approx. Wall SF: 0	Ceiling/Floor SF:	0

Spec #	Spec	Quantity	Units	Unit Price	Total Price

Trade:	9	Environmental Rehab

4	9001

Lead Scope of Work

See attached lead scope of work.

1.00	AL

Address: 4401 Gatewood Drive	Unit:  Unit 01

Location Total:

Unit Total for 4401 Gatewood Drive,:		 Address Grand Total for 4401 Gatewood Drive:

Bidder:

Scope of Work for Lead Hazard Control

# 4401 Gatewood Drive	Date 6/2/26

All work described must be performed by a North Carolina state certified lead firm and a state lead permit is required. All work must comply with all applicable federal, state, and local regulations. Provide a separate price for each line item and add the total of line items at the end of the scope. Transfer the lead scope total to the lead control line item in the main scope of work.

|  | Method of Control and Scope of Work | Number | Unit Cost | Line-Item Cost |
| --- | --- | --- | --- | --- |
| Exterior |  |  |  |  |
|  | The contractor is responsible for all project requirements, including but not limited to: Obtaining all permits required. Said permits shall include all items in this scope of work. Provide temporary toilet facilities from job start until the completion of work. Provide AS MANY roll-off dumpsters as needed without damaging the site. Collect construction debris using dust control methods. Remove dumpsters and repair any evidence of the dumpster’s use. Contractor may haul debris away daily using dump trailers or trucks. |  |  |  |

| Side A through Side D – light brown and brown wood window casings, headers, sills, sashes and wells and light brown and brown wood and metal lintels | Replace wood window sashes and wells with vinyl windows and cover window casings, headers and sills with Tyvek and aluminum |  |  |  |
| --- | --- | --- | --- | --- |
| Side A – light brown wood entry ceiling/overhang, crown moldings and trim boards and brown metal ceiling support columns | Cover with Tyvek and aluminum or vinyl. |  |  |  |
| Door A1 (to Room 1) – brown wood door casings, header, lintel and threshold | Replace threshold and cover door casings, header and lintel with Tyvek and aluminum. |  |  |  |
| Door 1C1 (to Room 2) – brown wood door casings, header, lintel and threshold | Scrape loose paint and re-paint. |  |  |  |
| Lower Level Door C1 (to Room 3) – light brown/brown wood door casings and header and light brown/brown metal lintel | Replace threshold and cover door casings, header and lintel with Tyvek and aluminum. |  |  |  |

Total Bid for Lead Scope $