# Invitation to Bid HNS 26-9

## NOTE: Contractors are not authorized to visit the property before or after the bid walk, without being accompanied by the City’s Rehabilitation Specialist.

Documents included in Package:

- Instruction to Bidders

- Specs by Location/Trade (Scope of Work)

- Subcontractor Certifications (if applicable)

- Floor Plan / Site Drawing (if applicable)

## Bid Walk & Bid Opening:

| Project Address: 7700 Heatherdale Ct – Safe Home |  |
| --- | --- |
| Bid Walk: MARCH 27, 2026 @ 9:30 AM |  |
| Bid Opening: APRIL 6, 2026 @ 2:00 PM |  |
| Client Name: Edward and Barbara Bell |  |
| Project Manager: Devin Smith | Contact Number: (704) 589-2130 |

Bid Walk and Bidding Instructions:

All bid walks are mandatory.

If you are going to be late the policy is the following:

Contact me BEFORE the start time if you are going to be late. If you are going to be more than 10 minutes late, we will proceed without you and you will not be permitted to bid.

The day of a bid walk the best way to reach me is at   (704) 589-2130

Bids must be received by the date, time and place specified. All others will be considered non responsive and disqualified.

# The Bids will be opened at 600 East Trade St. immediately following the above due date and time.

Company Acknowledgement: The undersigned, having become thoroughly familiar with the terms, conditions, limitations, and provisions of the housing improvement work to be performed at _7700 Heatherdale Ct__ to be funded through the City of Charlotte Neighborhood & Business Services, in addition, having fully inspected the site in all particulars, hereby proposes and agrees to fully perform the work within the time stated and in strict accordance with the proposed contract documents including furnishing of any and all labor and materials, and to do all work required to complete said Work in accordance with the advised respective contractual, for the sum of money: All labor, materials, services and equipment necessary for the completion of the Work shown on the Drawings and in the Specifications: Dollars_($ )_ Written total

Specs Dated: 3/12/2026 Number of Pages: 5 Total

Addenda # 1 Dated: Number of Pages:

Addenda # 2 Dated: Number of Pages:

Project Schedule: Minimum Start Date -

Completion Deadline:

Please Print and Sign:

Company Name/Firm:

Authorized Representative Name:

Signature: Date:

Requirements For Bidders

The City awards rehabilitation bids to the lowest responsive and responsible bidder. A responsible bidder for the safe home program is one who:

- Is a licensed general contractor in the State of North Carolina

- Has a Renovate, Repair & Paint Certification (http://www2.epa.gov/lead/renovation-repair-and-painting-program)

- Is not listed on a local, state or federal debarment list

- Carries an appropriate amount of insurance

- Can provide references verifying the contractor has completed work of a similar scope in a good workmanship like manner or successfully completed prior work for the Safe Home program. Referenced work must have been completed in one year or less from date of this invitation to bid.

A responsive bidder must:

- Submit all requested documentation on time

- Meet the above requirements for responsibility at the time of bid submittal

- Have the capacity to meet the required schedule for the project

- Existing rehab projects contracted by the contractor must be on schedule

The City reserves the right to waive any minor informalities or irregularities, which do not go to

the heart of the bid submittal or prejudice other offers, or to reject, for good and compelling reasons, any or all bid submittals.

Rehabilitation Specialist – Devin Smith City of Charlotte

Neighborhood and Business Services 600 E. Trade St.

Charlotte, NC 28202

PH: ( 7 0 4 ) 5 8 9 - 2 1 3 0

Fax:

# Instructions to Bidders

## Explanations to Bidders

Any explanation desired by a Contractor regarding the meaning or interpretation of the advertisement for bids, drawings, specifications, etc., must be requested in writing to the Rehabilitation Specialist with sufficient time allowed for a reply to each Contractor before the submission of their bids. Any interpretation made will be in the form of an addendum to the invitation for bids, drawings, specifications, etc., and will be furnished to all prospective Contractors. The Contractor must acknowledge any revision to the bid documents in the space provided on the bid form and it must be submitted with their bid package.

## Preparation of Bids

Bids shall be submitted on the forms furnished, or copies thereof, and must be manually signed. If erasures or other changes appear on the forms, the person signing the bids must initial each erasure or change

- The Contractor’s Bid Proposal must be properly executed and submitted on the form provided. Bids by Contractors must be received by the Rehabilitation Specialist at the time and place specified on the “Invitation to Bid”

- No bid will be considered unless all individual work items on the Bid Form including any addendums are priced. The sum of all work items must equal to the Total Bid Amount.

- Unless called for, alternate bids will not be considered

- Modification of bids already submitted will be considered if received at the office designated in the invitation for bids by the time set for opening of bids

## Submittal of Bids

- Sealed bids will be submitted to the Safe Home Program Office as directed in the invitation to bid prior to or at the appointed bid opening time

- Bid will be time stamped on the date of delivery

- Each bid must be submitted in a sealed envelope bearing on the outside the name of the Contractor, the name of the project for which the bid is submitted, and the date and time of the bid opening

- All bids submitted must be typed or written in ink and signed by the Contractor's designated representative

- Failure to include all forms may result in rejection of a bid

- Required Bid Package Forms:

  - Scope of Work

  - Addenda Acknowledgement

  - Itemized Work Sheet

  - Subcontractor Certifications (if applicable)

## Bid Validation

- All approved bids will be valid for 60 days after the bid opening date

HOUSING AND NEIGHBORHOOD SERVICES

http://housing.charlottenc.gov | 600 E. Trade Street | Charlotte, NC

City of Charlotte Housing and Neighborhood Services

600 E. Trade Street Charlotte, NC 28202

704-336-7600

Address: 7700 Heatherdale Court	Unit:  Unit 01

Location:	1 - Bathroom	Approx. Wall SF: 0	Ceiling/Floor SF:	0

Spec #

3/12/2026

#### Spec	Quantity	Units	Unit Price	Total Price

| Trade: | 22 | Plumbing & Fixtures |  |  |
| --- | --- | --- | --- | --- |
| 1 | 0206 | Lavatory Faucet |  |  |
|  |  | Replace both bathroom faucets with new metal faucets, drains with pop-up, p-traps, supply lines, full port ball type shut-off valves and escutcheon plates on all supply and drain lines. Owner choice of style and finish. |  |  |
| 2 | 0212 | Miscellaneous Plumbing Scope |  |  |
|  |  | Repair leaking tub drain line serving the main floor bathroom. Remove and replace the entire basement bathroom ceiling below (new drywall, finish, and paint). |  |  |
| 3 | 0214 | Replace Tub/Shower Valve and Trim |  |  |
|  |  | Remove the existing tub and shower valve and trim and replace with new per the Plumbing Code and the Construction Standards. Owners choice of style and finish. |  |  |
|  |  | Access to the valve will be through the back side of the tub wall. DO NOT REMOVE TUB SURROUND FINISH MATERIALS. |  |  |
|  |  | Scope includes patching and painting on the back side of the wall to match the existing wall surface. |  |  |
| 4 | 0224 | 17" Height Commode Replace |  |  |
|  |  | Install a 17" height, 2 piece, close coupled, white, vitreous china commode with a maximum water usage per flush of 1.6 gallons. Include plastic or pressed wood white seat, supply pipe, shut-off valve, flap valve and wax seal. Repair any leaks below. |  |  |
|  |  |  | Location Total: |  |

Location:	2 - Electrical	Approx. Wall SF: 0	Ceiling/Floor SF:	0

Location:	2 - Electrical	Approx. Wall SF: 0	Ceiling/Floor SF:	0

#### Spec #	Spec	Quantity	Units	Unit Price	Total Price

| Trade: | 23 | Electrical |  |
| --- | --- | --- | --- |
| 5 | 0178 | Smoke and Carbon Monoxide Detectors--Battery Powered |  |
|  |  | Install UL approved ceiling mounted battery powered smoke and heat detectors in all bedrooms and outside of all sleeping areas where they are currently missing. |  |
|  |  | Detector in the hall way shall be a combination CO/smoke detector. |  |
|  |  | Installation shall comply with all requirements of the Electrical Code SMOKE AND CARBON MONOXIDE DETECTORS. |  |
| 6 | 0186 | REPLACE RECEPTACLES, SWITCHES AND COVER |  |
|  |  | PLATES |  |
|  |  | Replace all existing interior and exterior receptacles, switches, and cover plates throughout the entire house. |  |
|  |  | Replace all two prong receptacles with three prong receptacles. |  |
|  |  | Install GFCI breakers on all two wire receptacle circuits. |  |
|  |  | Install GFCI/AFCI breakers on all bedroom receptacle circuits. |  |
|  |  | Label outlets per Code requirements. |  |
|  |  | Receptacles in Code defined wet or damp locations shall be GFCI protected. |  |
|  |  | Scope includes repair of any wall or ceiling damage. Rewire as neccessary so all devices and fixtures function correctly. |  |
| 7 | 0189 | Electrical Scope |  |
|  |  | Terminate all open splices inside secured, closed junction boxes. |  |
| 8 | 0192 | REPLACE LIGHT FIXTURES |  |
|  |  | Replace up to four existing ceiling or wall-mounted light fixtures with UL-approved LED light fixtures that include shades and lamps. Install junction boxes where required and correct any unsafe wiring. |  |
|  |  |  | Location Total: |

Location:	3 - Kitchen	Approx. Wall SF: 0	Ceiling/Floor SF:	0

#### Spec #	Spec	Quantity	Units	Unit Price	Total Price

#### Trade:	20	Flooring

#### 0231	Kitchen Floor System Repair

Remove all fixtures built in and not built in. Dispose of damaged/unsafe components of floor assembly. Work may include installation of 2"x 8" joists hung 16" on center, plywood or tongue and groove subfloor. Include replacing all deteriorated band joists and insulating floor to code.

Location:	3 - Kitchen	Approx. Wall SF: 0	Ceiling/Floor SF:	0

#### Spec #	Spec	Quantity	Units	Unit Price	Total Price

#### Trade:	20	Flooring

#### 0241	Kitchen - Interlockking Vinyl Plank Flooring

Install waterproof interlocking vinyl plank flooring. Flooring shall have a minimum 25 year manufacturer’s warranty and shall be installed strictly according to manufacturer's specifications.

Contractor is responsible for preparing subfloor as necessary to satisfy manufacturer’s installation specifications.

Install suitable matching transition pieces where flooring meets other flooring materials.

Install painted or stained wood quarter-round at all perimeters to complete installation.

Provide home owner choice of three to five pattern selections from program approved manufacturers.

#### Trade:	22	Plumbing & Fixtures

- 6835	Kitchen Sink -- Double Bowl Complete

Install a 22 gauge 33" x 22" x 7" double bowl, stainless steel, self rimming kitchen sink including a steel, metal body faucet and side sprayer, rated at 2.0 GPM or less, with a 15 year drip-free warranty, grease trap, supply lines, full port ball type shut-off valves & escutcheon plates on all supply & drain lines. NOTE: All copper is to be soldered (no compression fittings) & all PVC fittings glued.

#### Trade:	37	Appliances-Cabinets-Accessories

- 0272	Base Cabinets

Replace base cabinets. Cabinets to be constructed of solid maple and plywood. No particle board allowed. See contractor's manual guidelines & specifications for full requirements. Includes safe removal and reinstall of all plumbing and electrical.

#### 0278	Kitchen Countertops

Remove existing counter tops and dispose of properly.

Measure cabinets for new tops and order built to size post formed laminate tops from a reputable top shop. Top shop shall apply all end caps and supply side splashes where cabinets end at walls. Top shop shall perform all sink cut outs.

NOTE: Do not buy counter top blanks at big box stores and cut them to size on site.

Where counter top sections join, such as corners, caulk joints and install connectors supplied by the top shop to join the tops together, taking care to maintain a tight and flush transition all along the joint.

Location:	3 - Kitchen	Approx. Wall SF: 0	Ceiling/Floor SF:	0

#### Spec #	Spec	Quantity	Units	Unit Price	Total Price

#### Trade:	37	Appliances-Cabinets-Accessories

Attach counter tops to cabinets with screws front and back at all top ends, both sides of sinks, and at least every 3’ in between.

If counter tops do not fit tightly to wall, trim drywall and/or backsplash margin as necessary to maintain a caulkable gap. Neatly caulk counter tops to adjoining walls with low VOC matching caulk.

Provide owner with a minimum of five pattern choices that coordinate well with the cabinets.

### Location Total:

Location:	4 - Exterior	Approx. Wall SF: 0	Ceiling/Floor SF:	0

#### Spec #	Spec	Quantity	Units	Unit Price	Total Price

#### Trade:	30	Exterior Carpentry & Wall Framing

#### 054	Exterior Siding Repair/Replacement

Remove existing damaged or deteriorated exterior siding material. Replace appropriate insulation if necessary. Install new siding material to match existing.

#### Trade:	33	Porch/Columns/Railings

- 087	Secure Handrails

Secure front entry stairwell handrails.

#### Trade:	34	Doors & Windows

- 099	Vinyl Window

Properly dispose of existing window units. Field measure, order, and install new Energy Star rated vinyl, double glazed LOW- E windows including half screens. Insulate rough opening. Install new extension jambs, casing and sill as needed, prime and paint to match existing. Exterior trim to be wrapped in PVC coated aluminum coil as needed. Repair all areas disturbed by removal and installation. This installation is to include repairing any damaged or rotted casing, header, framing (to include king studs, jack studs etc.) sub-floor, floor joists, band joists and sill. Any windows replaced in a bathroom must have tempered glass. Owners choice of style.

#### 0115	Sliding Glass Patio Door

Remove and dispose of existing door, frame and threshold. Install new Energy Star rated pre-hung exterior vinyl siding patio door with screen and locking hardware. Insulate cavity. New casing and shoe molding will match existing. This installation is to include repairing any damaged header, door framing, wall framing, sub-floor, floor joists, band joists, and sill.

### Location Total:

Location:	5 - Stairs	Approx. Wall SF: 0	Ceiling/Floor SF:	0

#### Spec #	Spec	Quantity	Units	Unit Price	Total Price

#### Trade:	33	Porch/Columns/Railings

#### 079	Exterior Deck

Demo and properly dispose of existing rear deck. Replace with new exterior deck, including steps and railings. Properly secure it to the house structure. Install per current building code with specified treated lumber. Size and deminsions to code. Prep and paint or stain owners choice of color.

### Location Total:

Location:	6 - Basement	Approx. Wall SF: 0	Ceiling/Floor SF:	0

#### Spec #	Spec	Quantity	Units	Unit Price	Total Price

#### Trade:	34	Doors & Windows

- 0111	Basement Bathroom and Bedroom Slab Door Interior Install a slab door to existing jamb, mortise in hinges and latch. Hole saw for the hardware. Include new hardware. Locking for

bedrooms and bathroom, passage for all others. Match existing

style and finish.

### Location Total:

Unit Total for 7700 Heatherdale Court, Unit Unit 01:		 Address Grand Total for 7700 Heatherdale Court:

Bidder: