# Invitation to Bid HNS 26-15

#### NOTE: Contractors are not authorized to visit the property before or after the bid walk, without being accompanied by the City’s Rehabilitation Specialist.

Documents included in Package:

- Instruction to Bidders

- Specs by Location/Trade (Scope of Work)

- Subcontractor Certifications (if applicable)

- Floor Plan / Site Drawing (if applicable)

#### Bid Walk & Bid Opening:

| Project Address: LEAD/HEALTHY HOME SAFEHOME 3901 Cindy Lane Charlotte NC 28269 |  |
| --- | --- |
| Bid Walk: June 11th 2026 @ 10:00 am |  |
| Bid Opening: June 18th 2026 @ 2pm |  |
| Client Name: Avery Gasden |  |
| Project Manager: Stephon Blanding | Contact Number: 704-622-1685 |

Bid Walk and Bidding Instructions:

All bid walks are mandatory.

If you are going to be late the policy is the following:

Contact me BEFORE the start time if you are going to be late. If you are going to be more than 10 minutes late, we will proceed without you and you will not be permitted to bid.

The day of a bid walk, the best way to reach me is at 	704-622-1685

Bids must be received by the date, time and place specified. All others will be considered nonresponsive and disqualified.

The Bids will be opened at 600 East Trade St. immediately following the above due date and time.

Company Acknowledgement: The undersigned, having become thoroughly familiar with the terms, conditions, limitations, and provisions of the housing improvement work to be performed at 3901 Cindy Lane Charlotte NC_28269 to be funded through the City of Charlotte Neighborhood & Business Services, in addition, having fully inspected the site in all particulars, hereby proposes and agrees to fully perform the work within the time stated and in strict accordance with the proposed contract documents including furnishing of any and all labor and materials, and to do all work required to complete said Work in accordance with the advised respective contractual, for the sum of money: All labor, materials, services and equipment necessary for the completion of the Work shown on the Drawings and in the Specifications: Dollars_($ )_ Written total

Specs Dated: 5/19/2026 Number of Pages: 5

Addenda # 1 Dated: Number of Pages:

Addenda # 2 Dated: Number of Pages:

Project Schedule: Minimum Start Date -

Completion Deadline:

Please Print and Sign:

Company Name/Firm:

Authorized Representative Name:

Signature: Date:

Requirements For Bidders

The City awards rehabilitation bids to the lowest responsive and responsible bidder. A responsible bidder for the safe home program is one who:

- Is a licensed general contractor in the State of North Carolina

- Has a Renovate, Repair & Paint Certification (http://www2.epa.gov/lead/renovation-repair-and-painting-program)

- Is not listed on a local, state or federal debarment list

- Carries an appropriate amount of insurance

- Can provide references verifying the contractor has completed work of a similar scope in a good workmanship like manner or successfully completed prior work for the Safe Home program. Referenced work must have been completed in one year or less from date of this invitation to bid.

A responsive bidder must:

- Submit all requested documentation on time

- Meet the above requirements for responsibility at the time of bid submittal

- Have the capacity to meet the required schedule for the project

- Existing rehab projects contracted by the contractor must be on schedule

The City reserves the right to waive any minor informalities or irregularities, which do not go to

the heart of the bid submittal or prejudice other offers, or to reject, for good and compelling reasons, any or all bid submittals.

Rehabilitation Specialist City of Charlotte

Neighborhood and Business Services 600 E. Trade St.

Charlotte, NC 28202 PH:

Fax:

Instructions to Bidders

#### Explanations to Bidders

Any explanation desired by a Contractor regarding the meaning or interpretation of the advertisement for bids, drawings, specifications, etc., must be requested in writing to the Rehabilitation Specialist with sufficient time allowed for a reply to each Contractor before the submission of their bids. Any interpretation made will be in the form of an addendum to the invitation for bids, drawings, specifications, etc., and will be furnished to all prospective Contractors. The Contractor must acknowledge any revision to the bid documents in the space provided on the bid form and it must be submitted with their bid package.

#### Preparation of Bids

Bids shall be submitted on the forms furnished, or copies thereof, and must be manually signed. If erasures or other changes appear on the forms, the person signing the bids must initial each erasure or change

- The Contractor’s Bid Proposal must be properly executed and submitted on the form provided. Bids by Contractors must be received by the Rehabilitation Specialist at the time and place specified on the “Invitation to Bid”

- No bid will be considered unless all individual work items on the Bid Form including any addendums are priced. The sum of all work items must equal to the Total Bid Amount.

- Unless called for, alternate bids will not be considered

- Modification of bids already submitted will be considered if received at the office designated in the invitation for bids by the time set for opening of bids

#### Submittal of Bids

- Sealed bids will be submitted to the Safe Home Program Office as directed in the invitation to bid prior to or at the appointed bid opening time

- Bid will be time stamped on the date of delivery

- Each bid must be submitted in a sealed envelope bearing on the outside the name of the Contractor, the name of the project for which the bid is submitted, and the date and time of the bid opening

- All bids submitted must be typed or written in ink and signed by the Contractor's designated representative

- Failure to include all forms may result in rejection of a bid

- Required Bid Package Forms:

  - Scope of Work

  - Addenda Acknowledgement

  - Itemized Work Sheet

  - Subcontractor Certifications (if applicable)

#### Bid Validation

- All approved bids will be valid for 60 days after the bid opening date

HOUSING AND NEIGHBORHOOD SERVICES

http://housing.charlottenc.gov | 600 E. Trade Street | Charlotte, NC

Address: 3019 Cindy Lane	Unit:  Unit 01

City of Charlotte Housing and Neighborhood Services

600 E. Trade Street Charlotte, NC 28202

704-336-7600	5/19/2026

Location:	1 - Bathroom	Approx. Wall SF: 0	Ceiling/Floor SF:	0

Spec #	Spec	Quantity	Units	Unit Price	Total Price

| Trade: | 23 | Electrical |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0149 | LIGHT FIXTURE WALL MOUNTED | 1.00 | EA |  |  |
|  |  | Install a UL approved wall mounted 4 bulb LED light fixture with shade and lamps above the vanity. |  |  |  |  |
| 2 | 0180 | Combination CO / Smoke Detector Hard Wired | 1.00 | EA |  |  |
|  |  | Install a hard-wired combination carbon monoxide and smoke detector with battery backup. |  |  |  |  |
| 3 | 0183 | Smoke Detector Hard Wired | 3.00 | EA |  |  |
|  |  | Install UL approved, interconnected, ceiling mounted smoke and heat detectors permanently wired into a receptacle box with battery backups in bedrooms and outside of sleeping areas. |  |  |  |  |
| 4 | Custom | bathroom exhaust | 1.00 | EA |  |  |
|  |  | install exhaust fan in hall bath. make sure it is vented to the outside. the vent should be in the soffit closest to the bathroom |  |  |  |  |

Location Total:

Location:	2 - Bathroom	Approx. Wall SF: 0	Ceiling/Floor SF:	0

| Spec # | Spec | Quantity | Units | Unit Price | Total Price |
| --- | --- | --- | --- | --- | --- |
| Trade: 22 | Plumbing & Fixtures |  |  |  |  |
| 5 0214 | Replace Tub/Shower Valve and Trim | 1.00 | EA |  |  |
|  | Remove the existing tub and shower valve and trim and replace with new per the Plumbing Code and the Construction Standards. |  |  |  |  |
| 6 0224 | 17" Height Commode Replace | 1.00 | EA |  |  |
|  | Install a 17" height, 2 pieces, close coupled, white, vitreous china commode with a maximum water usage per flush of 1.6 |  |  |  |  |

Location:	2 - Bathroom	Approx. Wall SF: 0	Ceiling/Floor SF:	0

| Spec # | Spec | Quantity | Units | Unit Price | Total Price |
| --- | --- | --- | --- | --- | --- |
| Trade: 22 | Plumbing & Fixtures |  |  |  |  |
|  | gallons. Include plastic or pressed wood white seat, supply pipe, shut-off valve, flap valve and wax seal. |  |  |  |  |
| 7 6962 | BATHTUB/SHOWER SURROUND--5' FIBERGLASS | 1.00 | EA |  |  |
|  | Install a 5', white Swan 3 piece, fiberglass shower surround. carefully fit around all faucet components, caulking all seams and penetrations. |  |  |  |  |

Location Total:

Location:	4 - Bathroom	Approx. Wall SF: 0	Ceiling/Floor SF:	0

| Spec # | Spec | Quantity | Units | Unit Price | Total Price |
| --- | --- | --- | --- | --- | --- |
| Trade: 35 | Wall Coverings/Ceilings/Trim |  |  |  |  |
| 8 0251 | PAINT ENTIRE ROOM | 1.00 | AL |  |  |
|  | Remove or cover hardware, fixtures, furniture, accessories, and other room contents that are not to be painted. |  |  |  |  |
|  | Scrape loose, peeling, cracked and blistered areas. Clean oil, grease, fungus, dirt, and dust from surfaces. Fill holes and cracks. Repair any remaining defects in surfaces as necessary to produce smooth paint ready surfaces. |  |  |  |  |
|  | Prime all materials with a stain killing primer. NOTE: Very old paint or other site conditions may require the use of a bonding primer to achieve a durable finished product. The contractor is responsible for choosing products and processes that ensure a durable finished product. |  |  |  |  |
|  | Topcoat with two coats of low VOC acrylic latex paint. Use semi-gloss paint in kitchens and bathrooms. Use flat or eggshell sheens (owner’s choice) in all other rooms. |  |  |  |  |
|  | Surfaces to be painted include all walls, ceiling, trim, and doors. Interiors of closet are not included unless so specified. Ceilings shall be painted the owner’s choice of ceiling white or the selected wall color. |  |  |  |  |
|  | Replace or uncover hardware, fixtures, furniture, accessories, and other room contents. |  |  |  |  |

Location Total:

Location:	6 - Exterior	Approx. Wall SF: 0	Ceiling/Floor SF:	0

Spec #	Spec	Quantity	Units	Unit Price	Total Price

Trade:	9	Environmental Rehab

9	9697

EXTERIOR-- REPLACE RAILING SYSTEM

After establishing any required ground containment with polyethylene sheeting, wet mist, remove, package in

1.00	LF

Location:	6 - Exterior	Approx. Wall SF: 0	Ceiling/Floor SF:	0

Spec #	Spec	Quantity	Units	Unit Price	Total Price

Trade:	9	Environmental Rehab

polyethylene sheeting and dispose of deteriorated lead-painted railing system. Install a 36" high wood railing with 1" x 1" pickets, 3" on center supported by preservative treated 4" x 4". Prep, prime and topcoat with premium acrylic latex.

THE RAILING LOCATED AT THE BREEZE WAY NEED TO BE RESTORED. thIS INCLUDES THE POST.

THE HANDRAILS LOCATED AT THE FRONT STEPS WILL ALSO BE REPLACED

Trade:	15	Roofing

10	Custom

roofing

Remove existing shingles and replace any deteriorated substrate with OSB. New shingles should be approved by homeowners. Use only architectural shingles

1.00	EA

Trade:	23	Electrical

| 11 | 8175 | FLOOD LIGHT--DOUBLE BULB | 2.00 | EA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Install a building mounted, double lamp, incandescent flood light. Owner's choice of an interior or photoelectric switch. Any exposed exterior wiring shall be run in conduit. |  |  |  |  |
|  |  | ON THE LEFT FRONT OF THE GARAGE AND RIGHT REAR OF THE HOUSE INSTALL LED FLOOD LIGHTS WITH MOTION SENSORS |  |  |  |  |

| Trade: | 30 | Exterior Carpentry & Wall Framing |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 12 | 044 | Power Wash Siding | 1.00 | EA |  |  |
|  |  | Vinyl Siding & trim. Remove stains and dirt from siding using high pressure water and detergent mix. Adjust pressure to eliminate damage to the wood grain. |  |  |  |  |
| 13 | 048 | Soffit and Fascia Repair | 1.00 | EA |  |  |
|  |  | Replace deteriorated soffit and fascia areas, caulk, prep and Paint. Match existing material, trims and color. |  |  |  |  |
| 14 | 049 | Vinyl Siding | 1.00 | EA |  |  |
|  |  | Replace all deteriorated exterior building components. Wrap home with Tyvek vapor/ infiltration barrier. Install vinyl clapboard siding including corners, door and window trim to complete installation. Owner's choice of siding color, exposure, and texture with 50-year warranty. |  |  |  |  |

Trade:	32	Roof/Chimney/Gutters/Downspouts

| 15 072 Downspouts & Gutter Repair | 1.00 | AL |  |  |
| --- | --- | --- | --- | --- |
| Repair downspouts and gutters as needed. |  |  |  |  |
| Some ares of the roof is missing the gutters. Reinstall gutters and attach downspouts as needed |  |  |  |  |

Location:	6 - Exterior	Approx. Wall SF: 0	Ceiling/Floor SF:	0

Spec #	Spec	Quantity	Units	Unit Price	Total Price

Trade:	33	Porch/Columns/Railings

| 16 | 084 | Deck Boards FRONT PORCH | 1.00 | EA |
| --- | --- | --- | --- | --- |
|  |  | Remove and Dispose of Exterior Deck |  |  |
|  |  | THE EXISTING DECK BOARD ON THE FRONT PORCH SHOULD BE REPLACE. THIS IS A HAZARD FOR THE HOMEOWNER AS THEY ENTER THE RESIDENCE |  |  |

Location Total:

Location:	7 - Kitchen	Approx. Wall SF: 0	Ceiling/Floor SF:	0

Spec #	Spec	Quantity	Units	Unit Price	Total Price

Trade:	22	Plumbing & Fixtures

| 17 | 6815 | FAUCET--KITCHEN DUAL CONTROL | 1.00 | EA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Install a metal kitchen sink faucet with spray, dual controls, 15-year drip-free warranty and maximum flow of 2 gallons per minute. |  |  |  |  |

Location Total:

Location:	8 - General Requirements	Approx. Wall SF: 0	Ceiling/Floor SF:	0

Spec #	Spec	Quantity	Units	Unit Price	Total Price

Trade:	20	Flooring

| 18 | 0228 | Resilient Flooring entire home | 1,200.00 | SF |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Install 25 year warrantied resilient floor covering per |  |  |  |  |
|  |  | manufacturer's specifications. Flooring material is to be vinyl |  |  |  |  |
|  |  | interlocking planks and be waterproof. Include transitions and |  |  |  |  |
|  |  | painted or stained wood quarter-round at all perimeters to |  |  |  |  |
|  |  | complete installation. |  |  |  |  |

Trade:	35	Wall Coverings/Ceilings/Trim

19	0250

Prep & Paint Entire Interior

Remove or cover hardware and accessories not to be painted. Scrape loose, peeling, cracked and blistered areas. Clean oil, grease, fungus, dirt and dust from surfaces. Fill holes and cracks. Prime all new materials and spot prime existing with acrylic latex. Top coat with owner's choice of low VOC acrylic FLAT latex in all habitable rooms and low VOC acrylic

SEMI-GLOSS latex in kitchen and baths. Replace or uncover hardware, fixtures and accessories. Any moving of furniture required shall be included

1,200.00	SF

Location Total:

Unit Total for 3019 Cindy Lane, Unit Unit 01:

Address Grand Total for 3019 Cindy Lane:

Bidder:

# Scope of Work for Lead Hazard

Address :  3901 Cindy LN	Date: 05/19/2026

All work described must be performed by a North Carolina state certified lead firm and a state lead permit is required. All work must comply with all applicable federal, state, and local regulations. Provide a separate price for each line item and add the total of line items at the end of the scope. Transfer the lead scope total to the lead control line item in the main scope of work.

| Lead Hazard (from LIRA) | Method of Control and Scope of Work | Number | Unit Cost | Line Item Cost |
| --- | --- | --- | --- | --- |
| Contractor requirement | The contractor is responsible for all project requirements, including but not limited to: Obtaining all permits required. Said permits shall include all items in this scope of work. Provide temporary toilet facilities from job start until the completion of work. Provide AS MANY roll-off dumpsters as needed without damaging the site. Collect construction debris using dust control methods. Remove dumpsters and repair any evidence of the dumpster’s use. Contractor may haul debris away daily using dump trailers or trucks. |  |  |  |
| Exterior house |  |  |  |  |

| Window A2, Window B1, Window B2, Window C1, Window C6 and Window D1 – white wood window sashes and wells |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Exterior Garage |  |  |  |  |  |
| Window C1, Window C2 and Window D1 – white wood window sashes and wells |  |  |  |  |  |
| Door B1 (to Room 1) – white door, jambs and stops |  |  |  |  |  |
| Specialized Cleaning |  |  |  |  |  |

Total Bid for Lead Scope $